User Manual

Telix for DOS

Version 3.5x

Telix_ User Manual

Version 3.5x for use with MS-DOS 3.1 or higher

Copyright

Copyright(c)1986-1996 deltaComm Development, Inc.

Portions Copyright(c) 1996 ELSA GmbH. All Rights

Reserved.  No part of this publication may be

reproduced, transmitted, transcribed, stored in a

retrieval system, or translated into any other

language in any form by any means without the

written permission of deltaComm Development, Inc.

Software License Notice

Your license agreement with deltaComm Development,

Inc., which is included separately with the

product diskettes, specifies the permitted and

prohibited uses of the product. Any unauthorized

duplication or use of Telix, in part or in whole,

in print or in any other storage and retrieval

system is prohibited.

Government Restricted Rights Legend

Use, duplication, or disclosure by the US

Government is subject to restrictions as set forth

in subparagraph (c)(1)(ii) of the Rights in

Technical Data and Computer Software clause at

DFARS 252.227-7013 and paragraph (d) of the

Commercial Computer Software _ Restricted Rights

at FAR 52.227 - 19.  Subcontractor/Manufacturer is

deltaComm Development, Inc., Post Office Box 1185,

Cary, NC 27512, USA.

Licenses and Trademarks

Telix, the Telix Logo, deltaComm Development,

Inc., the distinctive deltaComm logo, SALT, SIMPLE

and HostPlus_,  are trademarks of deltaComm

Development, Inc. All Rights Reserved. Microsoft

Windows is a registered trademark of Microsoft

Corporation.  Other product names, corporate

names, or titles used within this document may be

trademarks or registered trademarks of other

companies, and are mentioned only in an

explanatory manner to the owners' benefit, and

without intent to infringe.

This document was created using Microsoft Word for

Windows 6.0

Printed in the United States of America

Table of Contents

BEFORE YOU BEGIN .............................................VII

Typographical Conventions ...................................vii

Keyboard Conventions ......................................vii

Mouse Conventions ........................................viii

TELIX CUSTOMER SERVICES .....................................VIII

Product Support Within the United States .....................ix

Electronic Services ........................................ix

Voice Telephone Support .....................................x

Product Support Worldwide .....................................x

WELCOME TO TELIX FOR DOS .......................................1

Program description ...........................................2

New Features of Version 3.5x ..................................3

The Telix Philosophy ..........................................3

Machine Requirements ..........................................4

Start-Up Directory ............................................5

GETTING STARTED ................................................6

Configuring Telix with MODEMCFG.EXE ...........................6

Starting a Telix Session ......................................8

INT14/FOSSIL Mode ..........................................11

Important Note for Network/Multi-tasking Operation .........11

Terminal Mode ................................................12

Command Entry ..............................................12

Sending Characters and Words with the Mouse ................13

The Help/Status Screen .....................................13

The Status Line ............................................14

Communications Parameters ....................................14

Entering and Editing Strings .................................15

Output String Translation ....................................16

Ending a Telix Session .......................................16

PROGRAM FEATURES ..............................................17

Hang-Up ......................................................17

Capture File .................................................17

Printer ......................................................17

Usage Log ....................................................18

Scroll-Back ..................................................20

iv  Table of Contents

Keyboard Definitions/Macros ..................................21

DOS Shell ....................................................23

DOS Command ..................................................23

Editor .......................................................24

DOS and File Functions .......................................24

Screen Image .................................................25

The Translate Table ..........................................26

Chat Mode ....................................................27

Clear Screen .................................................27

Local Echo ...................................................28

Add Line Feeds ...............................................28

Break Signal .................................................28

DOORWAY Mode .................................................29

Miscellaneous Options ........................................29

Sending Modem Strings ......................................29

Setting the Screen Size ....................................29

THE DIALING DIRECTORY .........................................31

The Display ..................................................31

Editing an Entry .............................................34

Adding Entries ...............................................34

Clearing Entries .............................................34

Finding an Entry .............................................35

Dialing ......................................................36

Automatic Queue Redial .....................................37

The Quick Dialing Bar ........................................38

Long Distance Codes ..........................................38

Other Functions ..............................................39

Inserting Blank Entries ....................................39

Deleting Entries ...........................................39

Revising Long Distance Codes ...............................40

Creating a Dialing Directory ...............................40

Loading a Dialing Directory ................................40

Printing the Dialing Directory .............................41

Sorting the Dialing Directory ..............................41

Moving Entries .............................................41

FILE TRANSFERS ................................................43

Downloading (Receiving) Files ................................43

Uploading (Sending) Files ....................................45

File Transfer Protocols ......................................47

Which One to Use .............................................51

Table of Contents    v

CUSTOMIZING TELIX .............................................53

Screen and Color Settings ....................................53

Terminal Options .............................................55

General Options ..............................................59

Modem and Dialing Settings ...................................62

Filenames and Path Settings ..................................66

ASCII Transfers ..............................................68

Protocol Options .............................................70

Kermit Transfers .............................................74

Comm Port Setup ..............................................74

INT14/FOSSIL Mode ............................................74

Language Setup ...............................................75

Saving Changes and Leaving ...................................75

TERMINAL EMULATION ............................................77

TTY ..........................................................77

ANSI-BBS .....................................................77

ANSI .........................................................78

VT102 ........................................................78

VT52 .........................................................79

AVATAR .......................................................79

USING SCRIPT FILES ............................................81

Creating and Compiling Scripts ...............................81

Executing a Script ...........................................83

SIMPLE .......................................................83

Learning a Script ............................................84

HOST MODE .....................................................87

APPENDICES ....................................................95

A Keyboard Command Summary ...................................95

B Troubleshooting ............................................97

C External Protocols ........................................109

Batch File Method .........................................109

Script File Method ........................................110

D File Formats ..............................................111

Index .......................................................112

Before You Begin  vii

Before You Begin

This Telix User Manual describes the operation and

use of Telix. For economy of words, this manual

will refer to Telix for DOS as simply Telix.

Typographical Conventions

This manual uses several standard notations

throughout its text to identify important

information.

Keyboard Conventions

All key names are shown in capital letters. For

example, the Control key is shown as CTRL. Your

keyboard's keys may not be labeled exactly as they

are in this manual.

Keys                Comments

Shortcut keys       Keys are frequently used in

combinations or sequences as

shortcut keys. For example,

CTRL-X means to hold down the

CTRL key while pressing the X

key; F10, F, O, means to press

and release each of these keys

in order.

RETURN/ENTER keys   These keys usually perform the

same action in Telix.  In this

manual, _Press ENTER_ means

that you can press either the

ENTER or RETURN key, unless

specifically instructed

otherwise.

viii Before You Begin

HOME, END, PGUP     These keys indicate that the

specific key is to be pressed.

Often these keys will be used

in combinations, such as CTRL-

END.

Mouse Conventions

-    If you have a multiple-button mouse, the left

mouse button is the primary mouse button

unless you have it configured differently

(for example, you may have Telix configured

to reverse the mouse buttons if you are left-

handed). Any procedure that refers to the

_left mouse button_ within this manual will

mean the primary mouse button, or the button

you normally press with your index finger.

_Right mouse button_ will always refer to the

secondary button.

-    _Point_ means to position the mouse pointer

until block cursor rests on whatever you wish

to point to on the screen.

-    _Click_ means to press and then immediately

release the mouse button without moving the

mouse.

Telix Customer Services

If you have a question about Telix, first look in

the printed documentation. We have tried to

anticipate all of your questions, and have

included the answers to many of them within these

pages. If you cannot find the answer in the

manuals, contact Customer Services.

Before You Begin   ix

Product Support Within the United States

Electronic Services

Assistance is available from several online

sources, 24 hours a day, 7 days a week.

CompuServe     Assistance is available on

CompuServe in the Telix section of the PC Vendor

Forums. In addition, the file libraries within

this forum contain a wealth of third party add-ons

for Telix, as well as the latest patches and fixes

for Telix. At any CompuServe ! prompt, type go

telix to access the PCVEN forum that contains the

Telix support section. You will need to select

_Telix_ from the message menu to access the Telix

message section and file library within this

Forum. An introductory CompuServe membership kit

is included with Telix.

Internet     Assistance is available by addressing

Email to _support@delta.com_.

Hobbyist Networks     deltaComm representatives

maintain a presence and offer basic support on the

following networks:  RIME, FidoNet, and ILink.

deltaComm Online     deltaComm Development, Inc.

maintains a 24 hour customer service bulletin

board that you can access with your modem at (919)

481-9399 or telnet to bbs.delta.com. This system

supports speeds of up to 28,800 bps with settings

of 8 data bits, no parity, and one stop bit.  Once

connected, you will find a large library of third

party add-ons, and will be able to address

questions directly to the Telix developers. In

addition, deltaComm Online offers our customers

many extra information services not related to

Telix.

x    Before You Begin

Voice Telephone Support

Unlimited no-service-charge support is available

from our customer service engineers by placing a

toll call between 9:00 A.M. and 6:00 P.M. Eastern

time, excluding holidays.

For Technical Support, call (919) 460-4556 to

speak to a software engineer. Before calling,

please make sure you have the following

information available:

-    Telix' version number from the ALT-Z screen

-    Your serial number from the Alt-6 command on

the program menu

-    The brand and model of your modem, and

whether it is internal or external

-    The exact wording of any error messages that

have appeared on your screen.

Product Support Worldwide

If you have purchased your copy of Telix outside

the United States, you may be able to obtain

support at one of several worldwide support

centers, either electronically or via voice.

Support availability and cost may vary from site

to site outside North America, and may require you

to have purchased your copy from the specific

support site to qualify for support from that

site.

Before You Begin   xi

Area      Contact

Australia &    My Computer Company

New Zealand    (02) 565-1044 BBS

(02) 550-4459 fax

(02) 565-1991 voice

Germany        Connect GmbH

(0241) 9177981 BBS

(040) 511 7074 fax

(040) 511 7073 voice

Great Britain  S.C.S.

(0493) 748904 voice

(0493) 748876 fax

Netherlands    The Owl's Nest

(035) 6028631 voice

(035) 6026527 fax

(035) 6023219 BBS

Welcome to Telix For DOS    1

Chapter 1

Welcome to Telix for DOS

Welcome! We know you are eager to begin using

Telix! If you are like most people, you want to

jump right in and start using your new program,

but we encourage you to at least skim through this

chapter first. This chapter will ease you into

Telix and have you placing your first call within

minutes.

One more thing before you start.... If you bought

your package from a dealer, send in your

registration card. Without it, we cannot tell you

about the next new widget we add to the program.

If you bought your copy directly from deltaComm

Development, Inc., you are already registered.

In This Chapter

-  Program Description

-  What's on the Screen

-  Program Requirements

-  Using the Help System

-  Installing Telix

-  Your First Connection

-  Starting Telix

2    Welcome to Telix for DOS

Program description

Telix is a full-featured communications program for PCs running

the PC/MS-DOS operating system, that is able to meet the needs of

almost any user. Telix features include:

-   An  extensive  range of  built-in  file  transfer  protocols,

including  Zmodem,  CompuServe Quick  B,  Xmodem,  Xmodem-1k,

Xmodem-1k-g,  Ymodem   (TRUE),  Ymodem-g,   Kermit,  SEAlink,

Telink, Modem7, and ASCII. Telix allows up to five 'external'

protocols to be defined and called from within Telix. In this

way, almost any kind of file transfer is possible from within

Telix.

-   Powerful  multiple dialing  directories, holding  up to  1000

entries, each containing many  pieces of information required

for  communicating  with remote  services,  such  as    phone

numbers and communications parameters.

-   Automatic queue redialing for hard-to-reach numbers.

-   Reliable DEC VT100/102 and VT52  terminal emulation. Keys may

be redefined by the user.

-   Full  access to  DOS including  a DOS  shell,  a DOS  Command

option, and full path support.

-   A Host Mode featuring  file transfers, password-protected DOS

shell, and two access levels.

-   A Chat Mode,  keyboard macros, a scroll-back  buffer, session

capture, usage log,  translate table, and a  variety of other

features, detailed in this manual. Above  all, Telix is fast,

and has a powerful, friendly user interface.

As a supplement to Telix, an extensive SALT script language is

available, which allows Telix to perform automated logons and

many more complicated functions. As an example of the power of

script files, the Telix Host Mode is written in the SALT script

language.

Welcome to Telix For DOS    3

New Features of Version 3.50

With version 3.50, Telix has been greatly improved and extended.

The most important additions are listed below:

-   Mouse-controlled   user  interface   with  pull-down   menus,

scrollbars in  listing windows and  in the  scroll-back mode,

and sending of words or characters by a mouse click.

-   Selectable language for the user interface, which can also be

changed during a Telix session.

-   Built-in INT14  mode for using  Telix within networks  (via a

modem  sharing software)  or with  FOSSIL  drivers (e.g.  for

passive ISDN boards).

-   Support  of higher  interrupts  (IRQ8  to IRQ15)  and  higher

addresses (above $7FFF)  for the COM ports.

-   Convenient editing and compiling of  script files from within

Telix via Script menu or hotkeys.

-   Extended  status  windows for  Zmodem  batch  file  transfers

(GSZ/DSZ compatible).

-   Telix now  provides CPU resources  for other  applications in

multi-tasking environments (such as Windows, OS/2 etc.).

The Telix Philosophy

When Telix was written, a basic idea was kept in mind. It was

felt that a powerful program could be simple to operate, with no

loss of speed in order to achieve this ease of use.

4    Welcome to Telix for DOS

Machine Requirements

Telix will run on an IBM PC, XT, AT, PS/2, or true compatible

with one hard drive and one floppy drive, or two floppy drives,

and with at least 380 KBytes of free memory. This is memory

available after DOS has been loaded. For a few functions, such as

the DOS Shell, more memory is helpful. Telix will support up to 8

defined comm ports (COM1 through COM4 are predefined), any number

of disk drives, and a printer connected to the parallel port.

To get full use of Telix your modem must meet certain standards

as well, especially for the Host Mode script. The Carrier Detect

signal should not be forced high (on) by the modem, so that Telix

can tell when you are online. This is usually controlled by the

setting of a DIP switch on the modem, or a modem command (often

&C1). The DTR (Data Terminal Ready) signal should not be ignored

by the modem (&D2 in many cases, or switch controlled).

Telix comes with a number of files and creates more in its normal

operation. Files included in the Telix package are:

TELIX.EXE       The Telix program itself

QDHOST.SLC      The QDHost Mode script file (compiled)

QDCONFIG.SLC    QDHost Mode configuration script (compiled)

*.KEY           The  various   key  definition   files  for   the

terminals supported by Telix

TELIX*.LNG      The various Language files  for the multi-lingual

user interface

TELIX.PIF       Program Information File for Microsoft Windows

Files (optionally) created and used by Telix include:

TELIX.CNF       The Telix configuration file

TELIX.LNG       The default Language file (after installation)

TELIX.FON       The default Dialing Directory file

TELIX.LD        The Long Distance Codes file

TELIX.CAP       The default session Capture File

TELIX.USE       The default usage log file

TELIX.IMG       The screen image save file

TELIX.KEY       The default user key macros definition file

TELIX.SWP       The temporary swap file for Telix during DOS

shell calls

QDHOST.CNF      The Host Mode configuration file

Welcome to Telix For DOS    5

If you have purchased the full version of Telix including the

script language compiler SALT, the disk will also contain the

following files:

CS.EXE          The SALT script language compiler

CSS.EXE         The SIMPLE script language compiler

QDHOST.SLT      The QDHost Mode script file (source)

QDCONFIG.SLT    QDHost Mode configuration script (source)

HOSTPLUS.DAT    Compressed HOSTPLUS advanced BBS

HOSTPLUS.DOC    The HOSTPLUS program reference manual

HINSTALL.EXE    The HOSTPLUS installation program

*.SLT           Several SALT source examples (e.g. log-on scripts

for different BBS systems)

MODEMCFG.EXE    The Telix modem configuration program

TELIX.MDM       Data file for MODEMCFG

Telix at one time or another can have a number of files open,

especially during Host Mode and a few other instances. If Telix

tells you it is having problems opening files then it is probable

that including the line

files=20

in your CONFIG.SYS file could clear up the problem. This allows

DOS to open more files (the default is files=8). For more

information on the CONFIG.SYS file see your DOS manual.

Start-Up Directory

When you run Telix it will remember the disk directory you ran it

from. After this point, even if you have changed the current

directory with the change directory (cd) command, Telix will know

that it should look for its program files on the start-up

directory. Some files will also be written on this directory

unless you specify another.

6    Welcome to Telix for DOS

If you set the environment variable TELIX, Telix will have

another (even better) way of knowing where to look for its files.

The command form at the DOS prompt is:

set TELIX=pathname

where pathname is a the legal DOS pathname of the directory in

which the Telix files are located. Do not leave any space between

the = and the arguments. For example, if you used set

TELIX=C:\TELIX\, Telix would know to look for its files in the

Telix directory on drive C. This way, no matter where you run it

from, Telix will always be able to find its files. To reset this

environment variable to nothing the command is:

set TELIX=

For more information on environment variables see your DOS

manual.

Getting Started

Configuring Telix with MODEMCFG.EXE

Telix comes with a complete installation program called

INSTALL.BAT that will walk you through a complete setup,

including creating or modifying your existing configuration to

work best with your modem; it will modify your modem settings to

work best with Telix. To run the modem configuration program at

any time after the installation (for example, when you install a

new modem), from the DOS prompt type:

MODEMCFG

The files TELIX.MDM and MODEMCFG.EXE must be in the same

subdirectory as TELIX.EXE for proper use. The TELIX.MDM modem

information file will be updated regularly on the support BBS's

listed in the Product Support section of this manual.  These

updates will  handle changes in modem strings, add new modems,

and will add new models as information about them reaches us.

Welcome to Telix For DOS    7

Throughout the menus of MODEMCFG.EXE, you may use the arrow keys

to move in a given direction, <Tab> and <Shift><Tab> to move

right or left, <Home>, <End>, <PgUp> and <PgDn> to move up or

down, or press the first letter of the menu item to move to the

next item beginning with that letter. A menu will be shown from

which you should select the manufacturer of your modem.  Under

each manufacturer is a list of their modems. Again pick the

appropriate modem.

If your modem is not listed, it is likely because the modem

manufacturer has not supplied the proper data for its addition.

Each modem is different, and for a modem to be listed, the modem

manufacturer must supply complete data about the command set and

functionality of the modem.  The modems listed as "Generic" or

"Unlisted" will usually work for modems that are not in the

current data file as posted on the support BBS's.

You will be shown a list of the settings for the modem itself,

including any switch settings if known. Please write down the

switch settings as you will need them later. When asked if you

wish to configure your modem, answer by pressing the <Y> key.

MODEMCFG will ask you for your comm port as well. Please refer to

your modem manual for this information.  If all goes well a

successful configuration message will be shown. If there is a

problem, ensure the proper comm port is specified and try again.

A similar setup procedure will take place for TELIX.CNF, the

configuration file for Telix itself. MODEMCFG will only change

the values pertinent to the performance of the modem in existing

configurations. Any existing color values or preferences will be

left intact. If TELIX.CNF does not exist, MODEMCFG will create

it.

The exit screen of MODEMCFG contains final instructions on how to

make best use out of the setup that has been written for you.

Please make a note of these for later referral.  You are now

ready for your first Telix session.

8    Welcome to Telix for DOS

Starting a Telix Session

To start the Telix program type the following while at the DOS

prompt:

Telix  [?] [B]  [D] [O]  [Q] [Cfilename]  [Ffilename] [Sfilename]

[COM] [INT14] [NOMOUSE] [TOFF]

The letters in the [square brackets] are optional command line

switches affecting what Telix will do upon start-up. The square

brackets should not be entered. The order and case of the options

does not matter but they must be separated by at least one blank.

?               This parameter  takes precedence  over all  other

parameters. It displays the possible command line

options of Telix without starting Telix itself.

B               Specifying 'B' in  the command line  forces Telix

to  use  the  PC's  BIOS  for  all  screen  write

operations and  keystroke handling,  whatever the

default setting  for these  operations might  be.

This is useful  when you want  to run Telix  in a

multi-tasking environment  such as Topview  or MS

Windows for just one session.  Note that Telix is

DESQview aware,  so you do  not have to  use this

mode for DESQview.

D               The  'D' option  forces  Telix  to drop  the  RTS

signal  during  disk  writes  for  those  systems

exhibiting  frequent   CRC  errors   at  rhythmic

intervals while downloading. This  is called slow

handshaking.

O               Specifying the  'O'  option  upon start-up  tells

Telix that you are already online (connected to a

host  system), and  that  it  should not  try  to

initialize the modem.

Welcome to Telix For DOS    9

Q               The 'Q' option in the  command line chooses quiet

mode for the current session.  When quiet mode is

on Telix will not sound any  alarms (e.g., when a

file  transfer  is  over),  no  matter  what  the

default setting for alarms is. This is useful for

those late  night sessions.  Any bell  characters

sent  by  the remote  system  are  still  sounded

however.

C               The   'C'   option   indicates   that   a   Telix

configuration  file   other  than   the  standard

TELIX.CNF should be used. If it doesn't exist, it

will  be  created. In  this  way,  you  may  have

multiple configurations  for Telix.  The name  of

the configuration file  should immediately follow

the 'C', and should include the extension.

F               The 'F' option indicates that a Dialing Directory

(*.FON)  file other  than  the default  TELIX.FON

should be  loaded  at start-up.  The name  should

immediately  follow the  'F'.  Note that  another

Dialing Directory  file may  also be  loaded from

within Telix (in the Dialing Directory screen).

S               The  'S' option  indicates  that  a  script  file

(explained elsewhere)  is to  be run  immediately

upon  start-up.   The  'S'  should   be  followed

immediately (no blanks) by the name of the script

file to execute.

COM             'COM'  forces  Telix  to  work  in  the  standard

communications mode (direct  communications via a

physical  serial  interface)   in  this  session,

regardless of  the setting  in the  configuration

file TELIX.CNF.

INT14           'INT14'  forces  Telix  to  work   in  the  INT14

communications   mode  (communication   via   the

interrupt 14h or a  FOSSIL driver), regardless of

the setting in the configuration file TELIX.CNF.

10   Welcome to Telix for DOS

NOMOUSE         'NOMOUSE' completely  disables the  mouse support

(default setting is on, if a Microsoft compatible

mouse driver has been  loaded and detected before

Telix is started).

TOFF            The 'TOFF' parameter forces Telix  not to provide

CPU time for other  applications in multi-tasking

environments (Windows,  OS/2). By  default, Telix

provides CPU resources.

XMS             Forces  Telix  to load  much  of  its  data  into

extended   memory.     If   extended  memory   is

available, then the memory  footprint of Telix in

main memory  will be about  40k less  than normal

when this switch is used.

When you start Telix for the first time, you will be asked to

select a language for the user interface. If no language files

(TELIX*.LNG) are found in the directory Telix is launched from,

an error is reported and Telix cannot be started. Telix will

create a copy of the selected language file and name it

TELIX.LNG. If you are sure that you will never use a different

language, you may then delete all .LNG files except for

TELIX.LNG. As long as the language files are present, you can

change the language at any time you wish.

Upon starting, Telix will display a title screen for a period of

25 seconds or until you press a key. Please read the message on

the title screen carefully. After you are past the title screen,

you will be in 'Terminal Mode' unless Telix can't find the

configuration file (usually TELIX.CNF), in which case it will ask

you a few questions about your machine and modem, and then will

write a configuration file for you.

Welcome to Telix For DOS   11

INT14/FOSSIL Mode

Unlike the standard communications mode, where Telix directly

communicates with an external modem or ISDN adapter via a serial

interface, Telix can also communicate via the BIOS interrupt 14h

in connection with a modem sharing program in a network or with a

FOSSIL driver (such as ATCAPI for passive ISDN boards). In this

case the modem sharing software or the FOSSIL driver must be

loaded before Telix is started. When creating the configuration

file during the first start, Telix will ask you which COM port is

to be used in INT14 mode. Specify the port used by your external

program or driver (in INT14 mode the virtual ports COM1 to COM4

are available). This setting can be easily changed later.

If Telix is started in INT14 mode (via the command line parameter

INT14 or the corresponding setting in the configuration file),

and no appropriate modem sharing software or FOSSIL driver has

been loaded before, Telix will report this and switch to the

serial INT14 routines of the PC BIOS. The bit rate in Telix will

be automatically set to 9600 bps in this case, since higher rates

are not supported by the BIOS. If you want to operate a modem in

this mode, disable the hardware handshake (CTS/RTS) of the modem,

as it is not supported as well. In general, the use of the BIOS

INT14 routines is not recommended because of their poor

performance and reliability.

While Telix is running, you can switch between the standard

communications mode (direct use of a serial port) and the INT14

mode at any time, if a suitable modem sharing program or FOSSIL

driver has been loaded before Telix was started. In INT14 mode

Telix will display INT14 in the status line (or FOSSIL, if a

FOSSIL driver has been detected).

Important Note for Network/Multi-tasking Operation

When Telix is used in a network or in a DOS multi-tasking

environment (such as DESQview) and the program SHARE.EXE is

active, "Sharing Violation Errors" may occur if several users try

to access TELIX.EXE at the same time. To prevent such errors, the

"Read Only" flag or the "Protectable" flag (Netware) of the

TELIX.EXE file should be set.

12   Welcome to Telix for DOS

Terminal Mode

Immediately after starting Telix, and for most of the time while

using it, you will be in Telix's Terminal Mode. While in this

mode any characters you type will be sent to the communications

port, and any incoming characters will be printed on the screen.

It is from the Terminal Mode that most Telix commands are

initiated.

Command Entry

Unless the mouse control has been disabled with the command line

switch NOMOUSE, all Telix functions can be accessed via mouse-

controlled menus. To use the mouse, a Microsoft compatible mouse

driver must be loaded before Telix is started. (Note: The

functions of the left and right mouse button can be reversed via

the Left-handed mouse function in the Miscellaneous menu.

However, we will refer to the primary mouse button as the "left"

button and the secondary button as the "right" button in this

manual. The term "clicking" always refers to the primary, i.e.

normally the left mouse button.)

The menu bar at the top of the terminal screen can be toggled on

and off with a double-click of the left mouse button, by pressing

<Alt><5> or selecting Menu line toggle in the Configuration menu

(the desired setting can also be permanently saved in the

configuration file). To select a function with the mouse, click

on the appropriate menu (e.g. Actions) and then on the desired

function (e.g. Hang-up modem) with the left mouse button. You can

also select menu items with the keyboard: Activate the menu bar

by pressing and releasing the <Alt> key and select the desired

menu and command by pressing the respective highlighted letter,

or move through the menus with the arrow keys. The highlighted

command is executed by pressing <Return>. Any menu can be exited

with <Esc>. The menu bar can be deactivated by pressing <Alt>

again (or <Esc>, if no menu is opened) or with a single mouse

click in the terminal screen. As long as the menu bar is active,

keyboard entries are not passed to the communications port.

Welcome to Telix For DOS   13

Some menu commands open additional menus, where you can select

fuctions or values with the mouse, with the arrow keys or by

pressing the highlighted letter, in the same way as described

above. Most commands can also be selected directly from the

Terminal mode by pressing the shortcut keys given behind the

respective menu item. For example, to access the Dialing

Directory, you would press <Alt><D>. That is, while holding down

the <Alt> key, you press the <D> key.

Sending Characters and Words with the Mouse

If the mouse control is enabled and a connection exists, clicking

with the left mouse button in the terminal screen (not in the

menu or status line) will send the word or character under the

mouse cursor to the communications port. If the menu item

Miscellaneous / Send word is checked (i.e. a checkmark appears

behind the menu item), the entire word is sent. If it is not

checked, only the character directly under the mouse cursor is

sent. Any string consisting of the characters a to z, A to Z, 0

to 9, the ASCII characters 128-165 (e.g. a, o, u) and $, #, &, @,

!, -, _, ~, \, ., :, * and ? is regarded as a word. All other

characters are regarded as word limits. If Send word is not

checked, any printable character is sent.  Clicking with the

right mouse button in the terminal screen during an existing

connection will always send a Carriage Return character

(<Return>, <Enter>, ASCII value 13).

The Help/Status Screen

While in Terminal Mode, selecting Help / General help or pressing

the <Alt><Z> sequence will switch you to a help screen showing

you a summary of all the available commands. If the key for a

command is pressed while the help screen is still on, the screen

will be turned off and the command will be executed. On the

bottom of the help/status screen is some information regarding

some current settings and the status of the current connection.

14   Welcome to Telix for DOS

Items included are: the current time and date, the connect

status, the elapsed time for the call in progress, the current

communications parameters, the state of the Capture File and

printer, the currently executing script file (if there is one),

the current Dialing Directory, the current keyboard macro

definition file, and a few other items.

Selecting the menu item Help / Command line options or pressing

<Ctrl><F1> will display a list of possible command line

parameters as described in section 2.2.

The Status Line

Telix has an optional status line which may be displayed at the

bottom or top of the screen. Its position can be selected under

Configuration / Preferences / Terminal options. The status line

can be toggled on/off by selecting Configuration / Status toggle

or pressing <Alt><8> in Terminal Mode. The status line displays

the current communications settings such as baud rate and parity,

the state of the Capture File, whether or not the printer is on,

the currently executing script file (scripts are explained

elsewhere), and whether Telix is Online or Offline. In INT14

mode, INT14 is displayed in the status line (or FOSSIL, if a

FOSSIL driver has been detected). During a file transfer,

information about the data flow control status (such as Flow or

Xoff) may appear in the status line as well.

Communications Parameters

Selecting Configuration / Comm parameters or pressing <Alt><P>

allows you to change the communications parameters. This is the

format in which Telix sends data over the modem, and expects to

receive it in. After pressing <Alt><P> a window is displayed with

the current parameters at the top, and the options in the rest of

the window. To change a value press the letter or number of the

setting you want, or click on the desired value with the mouse.

Welcome to Telix For DOS   15

There are a number of parameters that can be changed. The speed

is the bit rate (bps rate, bps = bits per second) that Telix uses

to communicate with your modem (sometimes also referred to as

baud rate, which is not quite correct). It should normally be set

to a higher value than the line bit rate of your modem (see also

Appendix C). All common bit rates up to 115,200 bps are

supported, however, for bit rates of 19,200 and higher, a fast

serial port with a 16550 UART chip is required (see also Appendix

C). Parity is a form of error checking. Allowable parities are

None, Even, Odd, Mark and Space. Data bits is the number of bits

(a bit is a one or a zero in a computer) in each character.

Allowable values are 7 and 8. Finally, the number of stop bits

parameter can be set to either 1 or 2. The most common format for

BBS use is 8 data bits, no parity, and 1 stop bit (8N1). On many

large networks such as CompuServe, the data format used is 7E1.

The communications parameters menu also allows you to select the

communications port you want to use. Note that by default only

the first four comm ports are defined. In INT14 mode, only four

(virtual) COM ports are available.

Important: Changing any parameters changes them only for the

duration of the current communications session. If you want a set

of parameters to be the default when you start Telix, select

Terminal options under Configuration / Preferences or <Alt><O>.

Entering and Editing Strings

In the submenus and dialog windows of Telix you will often have

to enter a string of characters, for example a filename, or a

list of numbers to dial. Telix gives you full editing powers when

you enter a string. As you type characters, you may use the

<Backspace> key to delete previously entered characters. The

<Left> and <Right> arrow keys allow you to move back and forth in

the string to edit what you have entered. By default, any

characters you enter overwrite any that are already there.

Pressing the <Ins> key will place Telix in insert mode, and any

existing characters are pushed ahead as you type. Pressing <Ins>

once more places you back in overwrite mode. <Home> and <End>

place the cursor at the beginning or end of the string,

respectively.

16   Welcome to Telix for DOS

Output String Translation

In quite a few instances while using Telix, the user must specify

a string to be sent out over the modem port. Sometimes this

string must contain characters which can not be printed on the

screen, for example, control characters, and even pauses. There

has to be a way of indicating these special characters, and it is

done through the use of special characters called translation

characters. When these translation characters are encountered in

an output string they are translated into other characters. The

default translation characters are:

^               When  this   character   (called   a  karat)   is

encountered in an output string  it will make the

following  character  a  control  character.  For

example, ^A will send out the code for <Ctrl><A>.

A few common control characters  are ^M, which is

the Carriage Return  character, ^J, which  is the

Line Feed character, and ^[,  which is the Escape

character. To  send out  the ^  character itself,

use two ^ characters, as explained above.

~               When  this   character   (called   a  tilde)   is

encountered it  will make Telix  pause for  1/2 a

second.  Note that  the  only  way  to send  a  ~

character out  is to use  the sequence  ^~. Using

two ~ characters in a row  will simply make Telix

pause twice.

Ending a Telix Session

To exit the Telix program, select File / eXit Telix or press

<Alt><X>, or double-click in the terminal screen with the right

mouse button. Telix will ask you if you really want to exit.

Press <Y> to confirm the exit, <N> or <Esc> to return to Telix.

If Telix detects that you are still on-line (connected to another

computer), it will also present a third option, Hang-Up. If this

option is selected, Telix will make sure that the connection is

broken before exiting. Attention: If you select Yes rather than

Hang-Up, the connection will remain even after leaving Telix!

Program Features   17

Program Features

Hang-Up

Selecting Actions / Hang-up modem or pressing <Alt><H> will make

Telix attempt to terminate the existing connection. Unless this

is disabled in the Configuration Menu, Telix first attempts to

hang up the modem by turning off a signal on the RS-232 port

called the DTR line. This works with most modems. If this doesn't

work, Telix then sends the hang-up string defined in the

Configuration Menu to the modem. If this still doesn't work and

Telix detects that you are still online, Telix will warn you. If

Telix is consistently telling you that hang-ups fail even when

they work, your modem is almost certainly overriding the 'Carrier

Detect' signal. See Appendix C for the solution to this.

Capture File

Telix allows you to capture (record) data coming in from the comm

port and printed on the terminal screen, to a disk file. To open

the Capture File, select File / Capture toggle or press <Alt><L>.

You will be prompted for the filename to save the log to. Press

<Return> to use the default filename. All data is added to the

end of the file if it exists already. To turn off the log, select

the menu item or press <Alt><L> again and select the Close

option. To temporarily pause capturing data to the file, select

the menu item or press <Alt><L> and select the Pause option.

While capturing is paused, any characters received are not saved

to the file. To then un-pause capturing, select the menu item or

press <Alt><L> and select the Unpause option.

Printer

Selecting File / Printer on/off or pressing <Ctrl><@> enables or

disables the Printer Log function of Telix. While the Printer Log

is active, information coming in from the comm port and printed

on the terminal screen is recorded on the connected printer as

well.

18   Program Features

Usage Log

Telix allows a usage log to be opened, which keeps track of calls

you make and files that you transfer. This is handy for keeping a

record of your long distance calls, for example. The Usage Log is

toggled on and off with File / Usage log toggle or the <Alt><U>

key. If it is currently closed, you are prompted for the name to

open it to (TELIX.USE is the default). If it is currently opened,

then it is closed. You may also set the Usage Log to be opened by

default at Telix start-up (under Configuration /

Preferences / General options).

Following is a sample portion of a log:

95-03-17  00:12:41  Telix Usage Log Created.

95-12-24  00:12:41  Telix Usage Log Opened.

95-12-24  00:13:06  Connected with : TBoard

95-12-24  00:13:06  ++ At phone # :241-0241

95-12-24  00:13:06  ++ Settings : 57600,N,8,1

95-12-24  00:14:53  Hangup command selected.

95-12-24  00:16:25  Connected with : Telix Support BBS

95-12-24  00:16:25  ++ At phone # : 1-919-481-9399

95-12-24  00:16:25  ++ Settings : 57600,N,8,1

95-12-24  00:17:17  Download using Zmodem protocol.

95-12-24  00:17:19  ++ File : D:\DL\ZTC100.ZIP

95-12-24  00:41:09  ++ Chars per second : 2566

95-12-24  00:58:28  ++ File : D:\DL\OPT100.ZIP

95-12-24  01:22:54  ++ Chars per second : 2612

95-12-24  01:23:11  Hangup command selected.

95-12-24  01:23:12  Exiting Telix.

Program Features   19

Lines that start with ++ are a continuation of previous lines.

Here is a listing of possible entries:

Menu command            Key            Log Entry

File / Usage log        <Alt><U>       Telix Usage Log Created.

Telix Usage Log Opened.

Telix Usage Log Closed.

Phone / Dialing Dir     <Alt><D>       Connected with: (entry)

++ At phone # : (phone #)

++ Settings : (settings)

File / Receive file     <Alt><R>       Download using (protocol)

++ File : (pathname)

++ Chars per sec : (CPS)

++ Transfer aborted.

File / Send file        <Alt><S>       Upload using (protocol)

++ File: (pathname)

++ Chars per sec : (CPS)

++ Transfer aborted.

Actions / Hang-up modem <Alt><H>       Hangup command selected.

File / eXit Telix       <Alt><X>       Exiting Telix.

The entries on the right are related to the commands shown on the

left. Note that a  script file has the ability  to stamp text

into  the   Usage  Log,  so   other  entries   are  possible.

20   Program Features

Scroll-Back

To review past screens of characters, select Actions / Scroll

Back or press <Alt><B>. You will be in the Scroll-Back display.

You will be able to move through the last one thousand to 64

thousand characters received (depending on the setting in the

configuration) by using the arrow keys to scroll line by line,

the <Home> and <End> keys to go to the beginning and end of the

buffer, and the <PgUp> and <PgDn> keys to scroll through the

buffer a page at a time. You can also scroll through the buffer

with the mouse: To scroll the text line by line, hold the left

mouse button down on one of the arrow symbols at the top or

bottom of the vertical scrollbar at the right screen border. To

scroll one page up or down, click on the scrollbar area above or

below the slider, or click directly on the slider and move it

with the mouse button held down.

To find a certain string in the buffer press <F>. Telix will

prompt for the string to search for. If the string is found the

line containing it will be placed at the top of the screen. To

search for the same string as last time press <Return> when asked

for the search string. The search is always performed in downward

direction, starting at the current cursor position.

To save the screen image to a file press <I>. Telix will prompt

you for the file to save the image to. If the file already

exists, the screen image will be appended to it. To save the

entire Scroll-Back buffer press <S>. To print out a screen image

to your printer press <Shift><PrtSc>.

All functions of the Scroll-Back buffer can also be selected

directly by clicking on the desired function on the bottom of the

screen with the mouse instead of pressing the shortcut keys

described above.

Program Features   21

Keyboard Definitions/Macros

Telix will let you assign text to a key so that it is sent to the

comm port when that key is pressed. This is usually called a

keyboard macro, and is useful in saving keystrokes (for example,

you can assign your name to a key). Telix actually keeps two key

definitions tables in memory at all times (each table holds the

text assigned to each key), the regular table and the terminal

table. The terminal table is used for terminal emulation key

assignments. When Telix is told to start emulating a certain

terminal, it loads into this table definitions specific to that

terminal. Therefore, for each terminal there is a key definition

file which is loaded as needed. The regular table on the other

hand is relatively constant. You might want to assign your name

to the <F4> key for example. This kind of definition would be put

in the user table.

When a key is pressed, Telix follows this procedure: If there is

a definition for that key in the terminal key table, send that

text; otherwise if there is a definition in the regular table for

that key, send the text; otherwise if the key represents an ASCII

value, send that value; otherwise if the key is a Telix command

(like <Alt><D>), perform that command.

The keyboard definition/macro editor is entered with

Configuration / Key defs./macros or by pressing <Alt><K> in

Terminal Mode. Telix will ask you if you want to access the

regular or the terminal key definition table. Unless you wanted

to modify the keys for a specific terminal, you would always

select the regular table. Telix will present the following menu:

Load    Save    Clear    Display    displayKey    Edit    eXit

Load            To load definitions  from a previously  saved key

definition  file  into memory,  select  the  Load

option. Telix  will ask for  the name of  the key

definition file  (and use  the extension  .KEY if

none is specified). Current  key assignments will

be cleared. The  standard key definition  file is

TELIX.KEY and is loaded at start-up if it exists.

22   Program Features

Save            To save  the current  key definitions  to a  disk

file, select the Save option. Telix will ask what

file to save them to, and  use the extension .KEY

if none is specified.

Clear           Selecting Clear  will clear  all the  current key

definitions in this table.

Display         Selecting Display will show what text is assigned

to each key.

displayKey      Selecting displayKey  will allow  you to  look at

what is assigned to one specific key, by pressing

it when prompted.

Edit            To actually enter or edit the  text assigned to a

key, select Edit.  Telix will print  the existing

definition and  allow you to  edit it.  Enter the

characters that  you want  the key  to send  when

pressed.  Control characters  and  pauses can  be

included  in   the  definition   in  the   manner

described previously  in the manual  under Output

String Translation. Remember that if you make any

changes they  will be  lost when  you exit  Telix

unless  you  use the  Save  option  as  described

above, to save them to disk.

Telix  also allows  you  to  run a  script  file  when a  certain

function key  is pressed. If a  function key definition has  an @

character as  its first character Telix  will assume the  rest of

the definition  is the  name of  a script file,  and will  try to

execute  it.   For  example  pressing  the   function  key  whose

definition is @logon would run the  script file called LOGON.SLC.

If  you really  want to  send  out an  @ character  as the  first

character in  the key definition, then  use two of  them instead.

E.g., the definition @@Hello would send out @Hello.

eXit            To leave  the  keyboard definition/macro  editor,

select eXit.

Program Features   23

DOS Shell

Selecting File / DOS shell or pressing <Alt><J> will let you jump

to a DOS shell. You will see the DOS prompt and will be able

execute any DOS command, for example dir. If there is enough

memory left, you can even run programs. There is an option

described later in the configuration section that allows you to

free up most of the computer's memory when shelling to DOS. To

exit the DOS shell type exit and press <Return>. You will be back

in Telix with the screen undisturbed from before your jump to the

shell. In order to run the DOS shell, there must be enough memory

left, and Telix must be able to find the file called COMMAND.COM.

If either of these requirements is not met, Telix will tell you

so. (The file COMMAND.COM is the DOS command interpreter. It is

usually pointed to by the DOS environment variable COMSPEC. See

your DOS manual for more information). If you run a serial port

related program in the shell, there is a good chance that it will

re-initialize the com port and confuse Telix. If this happens,

Telix will notice and re-initialize the port when you return from

the shell.

DOS Command

Selecting Actions / DOS command or pressing <Alt><V> allows you

to enter one DOS command or filename to run. This command is then

executed and control is returned to Telix. As can be seen this is

similar to the DOS shell, but is useful when you have only one

command to enter. There are many uses for this command. For

example, if your file viewing utility is called List, you could

press <Alt><V> and then enter List MYFILE.DOC. This would load

your file view utility and let you look at the MYFILE.DOC file.

After you terminated this program you would be back in Telix. Any

DOS command is legal, for example copy a:*.ZIP c:, or ren data

dat1. If for some reason you might want Telix to pause after the

command is finished, add a ; character to the end of the command

string, for example type MYFILE.DOC;. This stops the last screen-

full typed from scrolling by too quickly when control returns to

Telix.

24   Program Features

Editor

Selecting File / Run Editor or pressing <Alt><A> will allow you

to run your favorite editor from within Telix. In order to do

this you must supply the full name and path of your editor in the

Filenames and paths area under Configuration / Preferences. As

well, there must be enough memory left, and if you specified a

batch file to be run, Telix must be able to find the DOS command

processor COMMAND.COM.

DOS and File Functions

Selecting Miscellaneous / DOS functions or pressing <Alt><F> will

display a menu of some common DOS operations. For convenience, a

few commands available elsewhere have been included. The

individual functions and the letters to execute them are:

S:              Jump  to  DOS  Shell.  This   performs  the  same

function as File / DOS shell or <Alt><J>.

I:              Files Directory.  Telix will  prompt you  for the

optional  filespec, for  example,  *.DOC. If  you

want  to  list  all  the  files  in  the  current

directory then just press Return. Telix will then

do  one of  two  things. If  you  have defined  a

directory program  to use  (in the  Configuration

Menu), Telix  will run  it. Otherwise  Telix will

use its own internal  directory program, which is

similar to the DOS dir  command but also displays

the time  it  would take  to  transfer each  file

shown. The  time  is calculated  for the  current

default upload  file transfer  protocol. However,

the estimated  time  will only  be  correct if  a

connection exists and Telix knows the actual line

bit   rate,   i.e.  the   connection   has   been

established from  the Dialing  Directory and  the

modem has reported the correct bit rate.

Program Features   25

W:              Change Drive/Directory. The  current directory is

displayed and you  are asked to enter  a new one.

You can  enter a new  drive, a new  directory, or

both.  This will  become  the current  directory.

Note that this is slightly different from the DOS

cd command.

L:              Delete File. You are asked  to enter the filename

of the file to delete.

D:              DOS Command. This  performs the same  function as

Actions / DOS command or <Alt><V>.

E:              Run Editor.  This performs  the same  function as

File / RuN Editor or <Alt><A>.

Screen Image

Selecting File / Screen Image or pressing <Alt><I> saves an image

of the screen to the Telix Screen Image file. This file is

usually called TELIX.IMG, but can be renamed under

Configuration / Preferences / Filenames and Paths. If the file

already exists, the image is added to the end of it.

26   Program Features

The Translate Table

Sometimes it is necessary for certain incoming or outgoing

characters to be changed to another value or stripped altogether.

For this purpose, Telix keeps in memory an incoming and an

outgoing character 'translate table'. For every character in the

ASCII set, the table defines what it should be changed to. By

default, no character translation is performed, as for each

character, the new value is defined to be the same as the old

one. The translate table facility is entered by selecting

Configuration / Translate table or pressing <Alt><W> while in

Terminal Mode. Telix will ask whether you want to work with the

incoming or outgoing table, and then display the translate table

screen with the following menu at the bottom:

Load    Clear    Save    Edit    Other    eXit

Most of the screen is used to display translation values. Each

column holds on the left the original character (ASCII value),

and on the right the new character. Only half the ASCII set can

be displayed on one screen. Select the Other option to toggle the

display to show the other half of the character set.

Load            The Load  option  is used  to  load a  previously

saved translate table definition file. Telix will

ask for the filename, including extension.

Clear           The Clear option  is used to reset  all values in

the  translate  table  in   memory,  so  that  no

translation is performed.

Save            The  Save option  is  used  to save  the  current

translate table definition to  a disk file. Telix

will ask for the file name to  save the table to.

You  must  include the  extension  if  needed  (a

common extension for this purpose is .XLT).

Edit            The Edit  option  is used  to  actually edit  the

translate table. Telix will ask for the old ASCII

value, and  the new  one. If the  new value  of a

character  is defined  as  0,  that character  is

completely stripped.

Program Features   27

It is sometimes useful to have an incoming and/or outgoing

translate table load automatically when Telix is run. If the file

TELIXIN.XLT exists at Telix start-up, it is assumed to be a

default incoming character translate table, and is loaded into

that slot. Similarly, if the file TELIXOUT.XLT exists at start-

up, it is assumed to be the default outgoing translate table, and

loaded.

Chat Mode

Normally while using Telix with a remote host, your characters

are not echoed by Telix. When you type a character, it is sent to

the remote host, which checks it and then sends it back to you,

and only then is it printed on your screen. However if two users

want to chat with each other this becomes a problem, because each

user's software expects the other side to echo the characters

back, and since it doesn't, nothing is printed. Telix has a

special Chat Mode to deal with this.

When you select Actions / Chat Mode or press <Alt><Y>, Telix

enters Chat Mode. Any characters you type are printed on your

screen immediately, without having to be echoed by the other

side. As well, it takes only a <Return> to advance to a new line,

a Line Feed character is not needed immediately afterwards. Telix

also splits the streams of text. Any characters you type are

displayed on the bottom half of the screen, while the characters

coming from the remote side are displayed on the top half of the

screen. When you wish to exit Chat Mode, press the <Esc> key or

the left mouse button.

Clear Screen

To clear the screen, select Actions / Clear screen or press

<Alt><C>. The screen will be cleared to the default colors, and

the cursor will be placed in the upper left-hand corner.

28   Program Features

Local Echo

Selecting Actions / Local echo or pressing <Alt><E> will toggle

local echo on and off. If local echo is on, characters you type

are printed on the screen. If local echo is off characters you

type are not printed on the screen, they must be sent back to you

by the remote host (or your modem) to be printed on the screen.

Most hosts are set up so that local echo should be off; they will

do the echoing. This is called a full duplex host (this mode is

indicated by FDX in the status line). Often however when talking

to another user, or when talking to a half duplex host, local

echo must be turned on to see the characters you type (in this

case, HDX appears in the status line).

Add Line Feeds

When a line of characters is received from a remote system, it is

usually terminated by a Carriage Return followed by a Line Feed

character. If the Line Feed character is missing lines will

overwrite each other. If this seems to be happening when you are

connected to a remote host then you must make Telix add Line

Feeds to each incoming Carriage Return. Selecting

Actions / Toggle Linefeed or pressing <Shift><Tab> will toggle

the adding of Line Feed characters. If this option is on, every

time a Carriage Return is received, a Line Feed is added

immediately after it.

Break Signal

Some hosts require the use of a special modem signal called a

Break signal, to do certain tasks like end transmissions. To make

Telix send a Break signal to the remote host, select

Actions / Send BREAK or press <Ctrl><End>.

Program Features   29

DOORWAY Mode

Selecting Actions / DOORWAY mode or pressing <Alt><=> while in

Terminal Mode will toggle DOORWAY mode on/off. DOORWAY mode is

used by the DOORWAY utility by Marshall Dudley, and several other

applications. While in DOORWAY mode, when a key is pressed Telix

simply sends its raw scan code out the serial port, as returned

by the PC's BIOS keyboard function. Note that since for any key

except <Alt><=>, Telix just sends the value to the serial port,

you must first switch out of DOORWAY mode to perform any Telix

function with the keyboard.

Miscellaneous Options

Selecting Miscellaneous / Misc. functions or pressing <Alt><M>

while in Terminal Mode will cause Telix to display the Misc.

Options menu, which contains the following features:

Sending Modem Strings

To re-send the Modem Initialization string, select the first

option of the Misc. Options Menu. To send the Modem Auto-Answer

string, select the second option of the Misc. Options Menu. If

this string is properly defined, the modem will be placed in

auto-answer mode. The strings sent by these functions are defined

under Configuration / Preferences / Modem and dialing.

Setting the Screen Size

The third to fifth options of the Misc. Options Menu deal with

setting the screen size (height). These options are valid only on

EGA and/or VGA systems. Note that a standard EGA system can

switch only between 25 and 43 lines, and can not handle 50. Note

that some VGA cards can switch only between 25 and 50 lines, and

can not handle 43. Trying to use one of these options on a system

not equipped with an EGA or VGA video card is not allowed.

Inside The Dialing Directory   31

The Dialing Directory

Telix allows you to keep telephone numbers in files called

dialing directories. A Dialing Directory is variably sized and

can hold from 1 to 1000 entries. Besides the number, many other

fields of information are stored, such as the communications

parameters needed to talk to the service. All the fields are

described below. The default Dialing Directory file is called

TELIX.FON, but you can have as many directories as you want and

load them as needed.

To access the Dialing Directory, select Phone / Dialing Directory

or press <Alt><D> while in Terminal Mode. If the Use of Quick

Dialing bar option is enabled under Configuration / Preferences /

General options, a small window will appear, asking you to enter

the entries to dial or to press <Return> to see the Dialing

Directory. This first window is simply a very quick way to dial

your most commonly called numbers. In this case you must press

<Return> again to make the Dialing Directory itself appear.

The Display

Most of the screen is taken up by a window showing 15 (or more,

depending on the screen mode) of the possible 1000 entries in the

Dialing Directory.

32   Inside The Dialing Directory

On the bottom of the screen is a menu of options. This menu works

in a similar manner to the other menus found in Telix. In

addition, since dialing is such a common function, if any numeric

key is pressed, it is assumed that the List function is wanted

and this number is carried over as the first digit entered for

that function. To scroll through the directory one entry at a

time, use the <Up> and <Down> arrow keys. Use <PgUp> to go back

one page, and <PgDn> to go forward one page. Press <Home> to go

to the beginning of the directory, and <End> to go to the end of

the directory. It should be emphasized that whenever the line

Scroll with Up, Down, Home, End, PgUp, and PgDn is displayed, you

may scroll through the directory. You can also select an entry by

clicking on it with the mouse. A double-click with the left mouse

button on an entry directly executes the Dial function, even if

it is not highlighted in the menu.  For each entry, many items

are stored. All the fields can not be displayed at the same time;

you may toggle which fields are shown on the screen by selecting

the Toggle command. Each entry contains:

Name:           The name of  the remote service  (i.e. CyberCity,

CompuServe, ELSA ONLINE, etc.).

Number:         The phone number which Telix should dial to reach

the remote service.

Line Format:    The communications parameters (such as baud rate,

parity, etc.) needed to talk to this service.

Script:         A script file may be linked to  each entry in the

Dialing Directory.  When Telix  connects to  this

service, that  script file is  then run,  and may

for example  automatically log-on,  entering your

name  and   password  as   needed,  for   greater

convenience and speed.

LastCall:       The date of the last successful  call you made to

this remote service.

Total:          The total number of successful  calls you've made

to this number.

Terminal:       The terminal Telix should start  emulating when a

connection has been reached to this number.

Inside The Dialing Directory   33

P:              The first letter of the  Protocol which should be

set as the default if a  connection is reached to

this number.

E:              Whether local  Echo should  be on  or off  when a

connection is reached with this number.

L:              This controls whether the Add  Line Feeds setting

should be on or off when  a connection is reached

with this number.

H:              Whether  Telix   will   strip   the  High   (most

significant)  bit   of  incoming   characters  in

Terminal Mode.

BD:             Whether   a  received   Backspace   sequence   is

Destructive (that is, if the character backspaced

over should also be erased).

BK:             What  value  the  Backspace   Key  sends,  either

Backspace, or Del.

DP:             This (advanced)  setting  is used  to tell  Telix

what  Dialing Prefix  to  use  when dialing  this

entry.  The dialing  prefixes  are defined  under

Configuration / Preferences / Modem and  dialing.

They can  for example  be used to  dial out  of a

private branch exchange or to turn  on or off MNP

(error correcting)  support in  modems, to  avoid

delays when the number called does not support it

anyway.

Password:       This field  may hold  a password  for the  remote

service represented  by this entry.  The password

may be  used by  a script  file to  automatically

perform a log-on operation. Please note that this

field  is not  encrypted,  so  if security  is  a

problem,  do  not store  your  passwords  in  the

Dialing Directory!

34   Inside The Dialing Directory

Editing an Entry

To edit the currently highlighted Dialing Directory entry select

Edit from the Dialing Directory menu. A window will appear with

the old contents of the entry. You can move through the list with

the arrow keys, or click directly on a field with the mouse. To

edit the currently highlighted field, press <Return>, enter a new

text or select the desired value from the presented menu, and

press <Return> again. You may also move up and down in the list

of fields by using the <Tab> and <Shift><Tab> keys. Pressing

<Esc> one or more times will get you out of the edit window

without any changes being made.

When exiting the window, Telix will ask you if the Last Call and

Total Calls fields should be cleared. Answer Yes or No as needed.

If any data in one or more of the fields has been changed, Telix

will ask for confirmation before writing any changes to disk.

Adding Entries

To add a blank entry to the end of the Dialing Directory, select

Add from the Dialing Directory menu. A Dialing Directory may hold

up to 1000 entries.

Clearing Entries

To clear an entry or a range of entries select Clear from the

Dialing Directory menu. Telix will ask you to select the range of

entries to clear, by moving the highlight bar and pressing

<Return>. After confirming your choice, Telix will clear the

selected entries back to empty entries.

Inside The Dialing Directory   35

Finding an Entry

To search for a particular entry in the Dialing Directory select

Find from the Dialing Directory menu. You will be asked to give

the string to search for. If Telix finds the entry with that

string in it, that entry will be highlighted. If while searching

for the string Telix reaches the end of the directory, it will

wrap around to the beginning and continue the search. If the

string still can not be found Telix will say Sorry, string not

found. If you want to search for the same string as last time,

just press <Return> when asked for the search string.

36   Inside The Dialing Directory

Dialing

Telix allows you to dial a group of Dialing Directory entries

repeatedly until a connection is established, or you abort the

attempt. Selecting the Dial function from the Dialing Directory

menu or double-clicking on an entry with the left mouse button

will make Telix commence dialing in sequence all the currently

marked entries. An entry is marked or unmarked by moving the

highlight bar on it and pressing <Space>, or pressing the right

mouse button while the mouse cursor is on the currently

highlighted entry. If no entry is marked, the Dial function will

dial the currently highlighted entry.

You may also dial a user-entered list of entries, by selecting

the List function from the Dialing Directory menu. Telix will ask

you to enter up to 10 entries to dial in sequence (to make List

dialing even more convenient, you may also select the List

function by pressing a numeric key while in the Dialing Directory

menu; this number is then carried over and entered as the first

digit in the string you are asked to enter). While you are

entering the list of entries to dial you may also scroll through

the directory as described above. If you specify more than one

entry to dial, each should be separated by a space.

Selecting Phone / Queue redial #s or pressing <Alt><Q> while in

the Terminal Mode or selecting Redial from the Dialing Directory

menu will allow you to redial one or more numbers that you

entered previously using the List command, so that you do not

have to re-enter them. When using Phone / Queue redial #s or

<Alt><Q>, you will be shown the list and asked to edit it until

you are satisfied, and then to press <Return>.

Sometimes you may want to dial a number that is not in the

Dialing Directory. To do this select the Manual option of the

Dialing Directory menu, which stands for 'manual dial'. You will

be asked to enter a number to be sent to the modem. Whatever you

supply will be sent out exactly as it is, except for Long

Distance Codes (see below), which are still valid.

Inside The Dialing Directory   37

Automatic Queue Redial

When Telix has one or more numbers to dial, it redials repeatedly

until a connection is made. To keep you informed of the status of

the redial Telix displays a status window, and alerts you when a

connection is established.

Information included in the status window is the time redial was

started, the time the current attempt was started, the status of

the last attempt, the status of the current attempt, and a

countdown timer for the current attempt.

If you are dialing all the marked entries with the Dialing

Directory Dial command, when a connection is reached, the mark is

taken off the entry connected to, so that when you later select

the Dial function again, only the remaining entries are dialed.

If you are dialing a list of entries selected with the List

Dialing Directory command, when a connection is established the

entry connected to is taken out of the list (i.e. it is

unmarked), so that when you later select the Redial command from

the Dialing Directory or the Phone / Queue redial #s or <Alt><Q>

command from Terminal Mode, only the remaining entries will be

dialed.

While redialing, you have the option of changing the time that

Telix waits for a connection, and also the option of deleting a

number from the list to dial, by pressing T and D respectively.

Pressing <Esc> or the left mouse button will abort the current

dial attempt.

Normally, when a connection has been reached, Telix sounds an

alarm to alert you of this (if alarms are enabled under

Configuration / Preferences / General options). If a script file

is linked to the entry however, the alarm is skipped so that the

script may immediately proceed. It is thus the responsibility of

the linked script to alert the user of the fact that a connection

has been reached.

38   Inside The Dialing Directory

The Quick Dialing Bar

If the Use of Quick Dialing bar option is enabled under

Configuration / Preferences / General options, after selecting

Phone / Dialing Directory or pressing <Alt><D> first an inverse

bar is displayed on the bottom of the screen, asking you to

select the entries to dial or press <Return> to see the Dialing

Directory. This bar is a quick way to dial commonly called

directory entries whose numbers you have memorized. You may enter

a list of entries to dial just as if you had used the List

command of the Dialing Directory menu. This saves the Dialing

Directory from having to be displayed on the screen if you

already know the entry or entries you want to dial. The Quick

Dialing Bar can also be used to dial a number manually. To do

this just preface the number with an m. For example, m1-919-460-

4556 would dial 1-919-460-4556, just as if you had used the

Manual dial option of the Dialing Directory. Either lowercase or

uppercase m is accepted.

Long Distance Codes

A Long Distance Code is an advanced dialing option which adds

some flexibility. There are four Long Distance Code characters:

!, @,  $, and +, which when encountered while dialing expand to

user-defined strings.

If Telix is dialing a number from a Dialing Directory entry, and

one of the Long Distance Code characters is encountered in the

number, Telix will instead send whatever string has been assigned

to that code. For example, a Dialing Directory entry could

contain the phone number !967-1111. If the string 9,, was

assigned to the ! LD code character, Telix when dialing this

entry would first send the 9 followed by the commas, and then

send the rest of the digits. Refer to your modem manual for

information on what characters can be used in a dial string.

LD codes may also be used with the List dial function. When you

place one of them in front of or behind an entry number to dial,

it will be expanded to a string. For example you could have the

exclamation sign !, be expanded to 9,, or any string you specify.

Inside The Dialing Directory   39

You may place up to 3 Long Distance Codes in front and one Long

Distance Code after each entry number. For example, specifying

the entry +21! would send the string for the + Long Distance

Code, followed by the number from entry 21, followed by the

string for Long Distance Code !. The string

10  +15  20!

entered at the prompt would dial entry 10, then entry 15 with a

Long Distance Code in front of it, and finally entry number 20,

with a Long Distance Code after it. These numbers are redialed

repeatedly until a connection is reached, as explained above.

Other Functions

Selecting Other from the Dialing Directory menu will present a

menu of secondary functions, dealing with the deletion or

insertion of entries in the Dialing Directory, with revising Long

Distance Codes, and with the handling of dialing directories

(Telix allows you to have as many dialing directories as needed

and switch among them at will by loading the respective dialing

directory file).

Inserting Blank Entries

To insert blank entries in the Dialing Directory, select Insert

from the Dialing Directory Other menu. Entries will be inserted

at the current position of the highlight bar. Telix will ask how

many entries to insert and will ask for confirmation before

proceeding.

Deleting Entries

To delete an entry or a range of entries in the Dialing Directory

select the Delete option from the Dialing Directory Other menu.

The range to delete is selected by moving the highlight bar.

After confirming your choice, the entries will be deleted and any

following entries will move up to where the deleted entries were.

40   Inside The Dialing Directory

Revising Long Distance Codes

To revise the Long Distance Codes, select the option called

Edit_LD from the Dialing Directory Other menu. There are four

Long Distance Code characters: ! @ $ +. As described in the

section on Long Distance Codes on the previous page, when you

place one of them in front of or behind a number to dial, it will

be expanded to a string. After selecting this option you will be

shown a menu of the available Long Distance Codes. Selecting an

LD Code will let you modify the text currently assigned to it.

After you have made your changes, you must save them to disk with

the Save Changes option, if you want to use them every time you

run Telix.

Creating a Dialing Directory

To create a blank Dialing Directory (which can later be

modified), select the option called Create from the Dialing

Directory screen's Other menu. Telix will ask you for the name

the new directory should use. Any legal DOS filename is suitable,

and there is no default extension. Telix will then create the

blank directory on disk. The current Dialing Directory is not be

affected and will stay in memory.

Loading a Dialing Directory

To load another Dialing Directory select the Load option form the

Dialing Directory screen's Other menu. Telix will ask for the

name of the new directory and then load it into memory. Telix

assumes a default extension of .FON, so if the directory you want

to load does not have one, include a period at the end (e.g.,

TELIX.) The new directory becomes the default directory for the

rest of the session or until you load another one.

Inside The Dialing Directory   41

Printing the Dialing Directory

Selecting the Print option from the Dialing Directory screen's

Other menu allows you to print out the current Dialing Directory,

to the printer, the serial port, or any device or disk file.

Telix will ask you for the device or file to print to. Enter the

name or press <Return> to use the PRN device (printer). Since

these are DOS devices, Telix has no way of knowing, for example,

things such as whether or not your printer is online. So if Telix

seems to lock-up here, make sure your printer is on and it is set

to online.

Sorting the Dialing Directory

To sort the entries in the Dialing Directory select the Sort

option from the Other menu. You will be asked by which of four

possible criteria the entries should be sorted - System Name,

Telephone Number, Number of Calls, or Date of last Call - and if

they should be sorted in ascending or descending order.

Moving Entries

Select the Move option from the Others menu to move a range of

entries within the directory list. Use the highlight bar to

select the range of entries to be moved, and then indicate the

target position the entries should be moved to using the

highlight bar.

File Transfers   43

File Transfers

Sending files from one computer to another is one of the most

useful things you can do with your modem. Telix supports a full

assortment of the most popular file transfer protocols, including

Xmodem, Xmodem-1k, Xmodem-1k-g, Zmodem, CompuServe Quick B,

Kermit, Ymodem, Ymodem-g, SEAlink, Telink, Modem7, and ASCII.

Telix's file transfers are very fast, surpassing most programs in

speed by a respectable margin. All file transfers use a 10 Kbyte

write or read buffer, and full error detection and correction is

standard.

Telix allows you to define up to five external protocols, which

are completely integrated into the file transfer menus.  When you

select one of these protocols, Telix executes a DOS batch file or

Telix script file, which must then call the appropriate external

driver for the protocol. In this way, you may add virtually any

file transfer protocol to Telix. Adding external protocols is

defined in the chapter on customizing Telix.

Downloading (Receiving) Files

To start downloading (receiving) a file from a remote host, you

must usually inform the remote host of the files that you want it

to send to you before starting the transfer on the Telix side.

You must then start the download on the Telix side, unless you

have selected Zmodem as the transfer protocol and Zmodem auto-

downloads are enabled under Configuration / Preferences /

Terminal options. In this case the download starts automatically.

In all other cases, selecting File / Receive File or pressing the

<Alt><R> or <PgDn> key will display a menu asking which file

transfer protocol you would like to use.

44   File Transfers

If any external protocols are defined, they are also shown as

part of the menu.  If you have selected Xmodem, Xmodem-1k,

Xmodem-1k-g, or ASCII as the protocol, Telix will ask you for the

filename to save the file to. Telix always watches what you type

while online, and will often guess the name of the file you want

to transfer, in which case this name will be displayed and all

you have to do to accept it is press <Return>. Any legal DOS

filename is valid, including a path. If the filename you supply

already exists, Telix will ask you to confirm your choice or give

another filename. If you select any other protocol, the remote

sender of the file will supply the name to save the file to. If

you have entered a download directory name under

Configuration / Preferences / Filenames and Paths, Telix will

automatically save download files there if you don't specify

another path. Telix has automatic incoming file protection. If

the name is supplied by the host and exists already the file is

renamed by using numerals in the extension, as needed.

During the transfer, a status window of the following type is

displayed.  The status window shows the elapsed time, the number

of bytes received, and an approximate Character Per Second

rating, which can give an idea of the effective speed of the

transfer. Batch protocols (such as Zmodem) will also display the

expected file size and file transfer time. The displayed values

and the percentage bar always refer to the file currently being

transferred. If the Zmodem protocol of the remote host sends a

GSZ/DSZ or Telix 3.50 compatible extended protocol header and

several files are sent (batch transfer), a second set of

information and a second percentage bar are displayed to indicate

the overall amount of data transferred so far. For ASCII

transfers, a status window is not displayed, only a single status

line at the top of the screen. To abort a file transfer at any

time press the <Esc> key.

File Transfers   45

Uploading (Sending) Files

To start uploading (sending) a file to a remote host you must

also usually inform the host of the file you will be sending it

before starting the upload on the Telix end. To start the upload

on the Telix end, select File / Send File or press the <Alt><S>

or <PgUp> key. This will display a menu asking you which file

transfer protocol you want to use to send the file. After you

have selected the protocol, you will be asked for the file(s) to

send by a dialog box.

This is a complex dialog, and deserves detailed attention. Once

mastered, this dialog will greatly streamline the uploading

process. There are four separate windows and a menu with options

in the dialog. The selected window will have a bar at the top,

highlighting the name of the window. You may move from one window

to the next using <Tab> and <Shift><Tab> to move forward or

backward, respectively, or you can select a window with the left

mouse button. In any window, pressing <Up> Arrow or <Down> Arrow

move the highlight bar in that direction if possible. You may

also scroll through the lists with the mouse scrollbars on the

right window borders and select list entries with the left mouse

button. Pressing <F10> or clicking on Upload in the menu closes

the dialog and uploads the files selected.

Tagged Files:   This window contains  a list of all  of the files

Telix is  to upload.  It will  be blank  when the

dialog first appears. You may  select files to be

included in this  list as described  below. Files

already listed may be removed from this window by

highlighting the  file and  pressing <Return>  or

<Space> or clicking on the highlighted entry with

the mouse.

46   File Transfers

Path/Dir:       This  window is  where  the  currently  displayed

upload path will be displayed.  You may add files

to the  Tagged  Files List  from  this window  by

typing in the exact name of the file and pressing

<Return>. You may  use the wildcard  characters ?

and * (see  your DOS manual for  more information

about wildcard  characters). You  may change  the

currently displayed upload path by entering a new

path here.

Filelist:       This window contains  the files in  the currently

displayed upload path. This  window may contain a

maximum of approximately 700  files. Files listed

in this window  may be added to  the Tagged Files

List  by  highlighting  the   file  and  pressing

<Return> or <Space>, or clicking  twice on a file

with the left mouse button. If you wish to add or

remove all  files in this  window (both  those on

and off the  screen) to or from  the Tagged Files

List, press <F3>  or <F4> respectively,  or click

on the  corresponding  menu options  (Tag All  or

UnTag All).

Drives:         This  window contains  a  list  of all  available

disks in your  system, except for  floppy drives.

Users of removable disks  such as Bernoulli boxes

may have  to load a  cartridge to upload  a file.

You may change the drive on which the Filelist is

read from  by  selecting a  new  drive from  this

window.

NOTE            If Telix does not automatically  recognize one of

your drives, you may expressly specify the drives

to  appear  in the  list  under  Configuration  /

Preferences / General Options.

File Transfers   47

During the file transfer, a full status window is displayed,

unless you are using ASCII protocol, in which case only a status

line is displayed. Information shown is the expected time the

transfer will take, the time remaining, the number of bytes to

send, the number of bytes left, and an approximate Character Per

Second rating, which can give an idea of the effective speed of

the transfer. This information always refers to the file

currently being transferred. If several files are transferred

with Zmodem, a second set of information and a second percentage

bar are displayed to indicate the overall amount of data

transferred so far. To abort a transfer at any time press the

<Esc> key.

File Transfer Protocols

Xmodem          Xmodem  is  often   called  the   "lowest  common

denominator" protocol, and is the father protocol

of many others.  Xmodem sends files in  blocks of

128  characters at  a  time.  When the  receiving

computer is satisfied that  the block has arrived

intact, it  signals  so and  waits  for the  next

block. Error  checking is  a checksum  or a  more

sophisticated  Cyclic  Redundancy   Check.  Telix

supports  both  and will  automatically  use  the

right one. During a  download, Telix will attempt

to use the CRC, but will switch to checksum if it

seems the host  does not support CRC.  The Xmodem

protocol also needs a setup of  no parity, 8 data

bits,  and 1  stop  bit.  If your  communications

parameters  are  different from  these  when  you

start an  Xmodem transfer,  Telix will  switch to

the required setup for the  transfer, and back to

the original setup  after it is finished.  In the

Configuration Menu, you may enable Relaxed Xmodem

timing. This is for use with remote services that

can  not  tolerate strict  timing  during  Xmodem

transfers.

48   File Transfers

Xmodem-1k       This is a small variation of  the above that uses

blocks that are 1 Kilobyte  (1024 bytes) in size.

This protocol is still mistakenly called 'Ymodem'

by many programs, but people are gradually moving

over to the proper name.

Xmodem-1k-g     This is a  variation of the  above that  is meant

for error-free channels such  as error correcting

modems  or null  modem  cable  links between  two

computers.  It achieves  great  speed by  sending

blocks  one  after another  without  waiting  for

acknowledgment from the receiver. Because of this

however, it  can not  re-send blocks  in case  of

errors  (and  thus the  need  for  an  error-free

link). If an  error is detected by  the receiver,

the transfer is  aborted. This protocol  is still

mistakenly called  'Ymodem-g'  by many  programs,

but  people  are gradually  moving  over  to  the

proper name.

CIS Quick B     This  protocol is  used  only  on the  CompuServe

Information  Service.   It  is  quite   fast  and

specially  suited   for  the  networks   used  in

accessing  CIS,   which  have   large  turnaround

delays.  This  is  a   windowed  protocol,  which

basically means that unless  errors are detected,

data is being  sent at all  times. A CIS  Quick B

transfer  is  totally controlled  by  CompuServe.

After telling CIS what file(s)  to send, the user

does  nothing.  Telix,  in  Terminal  Mode,  then

detects the  command from CIS  which tells  it to

start  the  transfer  (note   that  CIS  Quick  B

transfer      must     be      enabled      under

Configuration / Preferences / Terminal   options,

or they will not work).

File Transfers   49

Zmodem          This  advanced protocol  is  very  fast and  very

reliable,  and  offers  many   features.  It  can

arguably be called  the most popular  protocol in

use today. Zmodem  can transfer a group  of files

in one  batch, and  keeps the  exact file  names,

sizes and  dates. Zmodem  can detect  and recover

from   errors   quickly,  and   can   resume   an

interrupted  transfer  at  a   later  time  (this

feature, called Crash Recovery, may be toggled in

the     Protocol      options     page      under

Configuration / Preferences). Zmodem is also very

well  suited  for  satellite   links  and  packet

switched networks.

SEAlink         SEAlink  is   an  advanced   version  of   Xmodem

developed by System Enhancement Associates. It is

a sliding  window protocol. SEAlink  is 15  to 25

percent  faster  than   regular  Xmodem.  SEAlink

passes  a  file's  name,   size,  and  date  when

transferring it,  and can  be used  to send  more

than one file at a time.

ASCII           In an ASCII transfer,  it is as if  the sender is

actually typing  the characters and  the receiver

is just recording them. There is no form of error

detection used. Usually  only ASCII files  can be

sent  in  this  way,   as  binary  files  contain

characters which  would foul  up the  screen. The

ASCII protocol  is used,  for example,  to upload

prepared messages to the message  editor of a BBS

instead of typing the text online.

Ymodem          This protocol  is  a variation  on Xmodem,  which

allows  for  multiple   files  to  be   sent  per

transfer. While transferring files,  it keeps the

correct name, size, and date, and  may use 128 or

1024 byte block sizes.

50   File Transfers

Ymodem-g        This protocol is a variation  on the above, which

achieves  very  high transfer  rates  by  sending

blocks  one  after another  without  waiting  for

acknowledgment. This however means than an error-

free link  such as  an error-correcting  modem or

null modem cable between two computers is needed.

If an  error  is detected  by  the receiver,  the

transfer is aborted.

Telink          This  protocol  is  mainly  found   on  Fido  BBS

systems.  It  is basically  the  Xmodem  protocol

using CRC checking with an extra block sent ahead

of the  file telling  its name,  size, and  date.

This protocol  allows more  than one  file to  be

sent at a time.

Modem7          Modem7 is  a close cousin  to Xmodem  that passes

the filename  before starting the  transfer, thus

taking away some of the user's work. It is common

on CP/M systems.  This protocol allows  more than

one file to be sent at a time.

Kermit          The  Kermit protocol  was  developed  to make  it

easier for computers  of different types  to send

files to  each other.  Almost any  computer using

Kermit can  be set  up to  send files  to another

computer using  Kermit (the same  can be  said of

Zmodem however).  Various options  in the  Kermit

protocol implemented in Telix  can be tailored to

the    user's   needs    under    Configuration /

Preferences,   option  Kermit   transfers.   This

implementation  of  Kermit  does  not  include  a

Kermit "server"  function, and  cannot use  SEND,

GET, or other server commands.

File Transfers   51

Which One to Use

Telix supports a large number of built in file transfer

protocols. To this can be added external protocols. It can get

confusing to decide which one to use. When both sides of the

connection only support one protocol, then obviously that is the

one to use, no matter what its deficiencies. Otherwise some

protocols are a better choice than others. On CompuServe, CIS

Quick B should be used. For most purposes, Zmodem is the best all

around protocol. It is very fast and very reliable. Ymodem-g and

Xmodem-1k-g are slightly faster. However these two protocols need

an error-free link. After that, Ymodem, SEAlink, and Xmodem-1k

are quite fast and pretty reliable. The last choice would be

Xmodem and its close cousins Telink and Modem7. ASCII transfers

are suitable only for text files and do not perform any error

detection or correction, so they should only be used as a

substitute for directly typed text (uploading a prepared message

to the message editor of a BBS).

Customizing Telix   53

Customizing Telix

Telix has many features which can be customized. These include

things like the screen setup, the communications parameters,

filenames and paths, and many other items. The default values for

these parameters are stored in a file called TELIX.CNF, the

configuration file. Telix will automatically read this file upon

starting up. The first time you use Telix, this file is created

for you with some standard default values. Selecting

Configuration / Preferences or pressing <Alt><O> while in

Terminal Mode brings up the Telix Preferences menu, which allows

you to change all these default settings.

The first eleven options are parameter screens, the last two deal

with leaving the Configuration Menu.

Screen and Color Settings

This screen allows you to set various video related options. You

will be shown a menu listing the various parts of the screen

displays (such as menu foreground colors). If you pick one of

these items you will be allowed to scroll through the possible

colors for it by using the arrow keys. Press <Return> to select

the currently highlighted color, or click on the desired color

with the mouse. If you want to leave the colors menu without

selecting one press <Esc> or click outside the colors menu.

54   Customizing Telix

Another setting on the Screen and color menu is the Screen write

mode. If you pick this option you will be asked in which way you

want Telix to write to the screen. This depends on the video

adapter present in your system. The first option is a direct

screen write. This is the fastest mode, and should be used if you

have a monochrome adapter, an EGA adapter, or another video

adapter that never gets snow. The second option is a direct

screen write with port checks. This method is slightly slower.

You should use this method if your screen gets snow on it or it

flickers while executing the DOS dir command. One card with which

you should use this is the color/graphics adapter. Don't use this

option if your video card doesn't need it, as it may interfere

with communications. The final method is a screen write with BIOS

calls. You should use this method with applications like Windows,

Topview or DoubleDOS. This method is quite slow however. Note

that you do not need this mode to run Telix under DESQview, since

Telix is DV aware.

Another setting on this menu is Default screen size. This allows

you to pick an alternate default screen size of 80x43 or 80x50.

Note that these modes require an EGA or VGA card. Note also that

many EGA cards can not do 80x50, and some VGA cards can not do

80x43.

When you have finished picking the colors, select the Keep

changes & exit option if you want to keep the color changes you

have made, or the Exit option if you want to discard them.

Customizing Telix   55

Terminal Options

This screen contains settings related to Telix operation while in

Terminal Mode (with some extending to other areas of Telix). To

select a setting to change, click on it with the mouse or type

the letter beside it. Telix will then allow you to change the old

setting. Press <Esc> at this point or click outside the menu to

exit without changing anything. Options include:

A:              Default Communications Parameters.  These include

comm port,  bps (baud)  rate, data  bits, parity,

and stop  bits.  Note that  the current  settings

will not change,  only the defaults for  the next

time you load  Telix. To change the  settings for

the current session  only, select Configuration /

Comm.  parameters  or  press  <Alt><P>  while  in

terminal mode.

B:              Emulate Before Translation. Some European systems

override  characters   used   by  many   terminal

emulations  to  signal color  changes  and  other

items  that  don't  show  up  on  the  screen  as

characters.  Normally  a  received  character  is

passed  through  the  translation  tables  before

being examined to see if it  is such a character.

This option allows the  emulation to occur first,

and  then only  translates  those characters  not

seen as emulation commands.

C:              Default Terminal  Type.  Allowable terminals  are

"TTY", "ANSI-BBS",  "VT102", "VT52",  "ANSI", and

"AVATAR". Note that the current terminal will not

be changed, only the terminal  used the next time

you run  Telix. To  change the  current terminal,

select   Configuration / Terminal  Emulation   or

press <Alt><T> while in terminal mode.

56   Customizing Telix

D:              Status Line.  Telix allows  a status  line to  be

displayed at the top or bottom of the screen. The

status line shows some  useful information. Valid

options  are  Top,  Bottom   or  None.  While  in

Terminal Mode the status line  may also easily be

toggled on  or off  by selecting  Configuration /

Status toggle or pressing <Alt><8>.

E:              Default Local Echo. This controls  whether or not

Telix should echo  on the screen any   characters

you type while in Terminal  Mode. This is usually

off for  BBS  use. Local  echo on  is often  also

called Half Duplex (HDX), while local echo off is

called Full Duplex (FDX).

F:              Add  Line Feeds.  This  controls  whether or  not

Telix should add Line  Feeds to incoming Carriage

Return characters while in  Terminal Mode. If you

are connected  to a remote  system and  lines are

overwriting  each  other,  this   should  be  on,

otherwise it should be off.

G:              Strip High  Bit. Telix can  strip the  high (most

significant) bit  from incoming data  bytes while

in  Terminal  Mode.  This   is  sometimes  useful

because it allows the high/parity bit of incoming

text to be ignored.

H:              Received  Backspace  Destructive.   This  setting

controls  how  Telix  treats  incoming  backspace

characters.   If    backspace   characters    are

destructive,  Telix backs  up  over the  previous

character and  erases it, otherwise  the existing

character is not erased.

I:              Backspace Key  Character.  Normally, Telix  sends

Backspace when  the <Backspace>  key is  pressed,

and DEL  when <Ctrl><Backspace> is  pressed. This

option allows these settings to be swapped.

Customizing Telix   57

J:              XON/XOFF  Software Flow  Control.  This  controls

whether  or not  Telix  should  use the  XON/XOFF

software handshaking  protocol while  in Terminal

Mode and during ASCII file transfers. This should

usually  be off,  as  most  systems nowadays  use

hardware  handshake.  However,  some  large  data

services  use  the characters  XON  and  XOFF  to

control data flow.

K:              CTS/RTS     Hardware    Handshaking.     Hardware

handshaking is  used with  high speed  modems and

null modem  cable links to  regulate the  flow of

data. If the modem  supports Hardware handshaking

this should be on. If a null modem cable is being

used and these signals are  valid over the cable,

this  should be  on.  If  CTS/RTS handshaking  is

enabled by default,  but the CTS signal  is found

off when Telix is started, Telix will report this

and wait  ten seconds  for CTS  to be  turned on.

Within these ten  seconds you can press  Space to

keep CTS/RTS  handshaking,  or any  other key  to

turn it  off.  If nothing  happens  in these  ten

seconds, the handshaking  is turned off  as well.

The same happens if the PC BIOS  is used in INT14

mode. In  this case,  Telix switches  to XON/XOFF

handshaking, since  CTS/RTS is  not supported  by

the BIOS routines.

L:              DSR/DTR     Hardware    Handshaking.     Hardware

handshaking is  used with  high speed  null modem

cable links  to regulate the  flow of data.  If a

null modem cable is being  used and these signals

are valid  over  the cable,  this  should be  on.

Often a null  modem cable will allow  one type of

hardware  handshaking to  be  used,  but not  the

other.

58   Customizing Telix

M:              Compuserve Quick B Transfers.  CompuServe Quick B

file transfers are initiated by CompuServe, which

sends  a few  special  characters (<Ctrl><E>  and

<Ctrl><P>).   Telix,    upon   detecting    these

characters  while in  Terminal  Mode, starts  the

transfer.  If  another  Host   is  sending  these

characters  and  causing  problems,  this  option

allows CIS Quick B recognition to be turned off.

N:              Zmodem Auto-Downloads.  Telix, while  in Terminal

Mode, can detect the packet from another computer

sending  files  using the  Zmodem  protocol,  and

automatically  begin  a Zmodem  download  on  the

local end. This option allows  this feature to be

disabled.

O:              Drop RTS during  Disk Writes. This  option forces

Telix to  drop the  RTS signal  line during  disk

writes  for  those  systems  exhibiting  frequent

errors at  rhythmic intervals  while downloading.

This  is  called slow  handshaking.  This  option

corresponds to the command line switch D.

P:              Answerback String. Some systems  require that the

terminal program  send  an identification  string

when an  ENQ (<Ctrl><E>)  character is  received.

You may define such  an 'answerback' string here.

If this  string  is empty,  nothing  is sent.  As

well, note that the answerback string is not sent

if  CompuServe  Quick  B  transfers  are  enabled

(above), as  <Ctrl><E> is  used as  part of  that

protocol.

Customizing Telix   59

General Options

This screen allows you to change some general settings,

including:

A:              Sound. If  this is  off Telix  will not  make any

sounds.

B:              Alarm Sound. If  this is off Telix  will not make

any alarm  sounds, but  will still  beep when  it

receives  the bell  code  from  the remote  host,

unless the Sound option is also off.

C:              Keep Aborted Downloads. If this is on, Telix will

keep files which were  aborted during a download.

Otherwise Telix will delete these files.

D:              Show  Free  Space  for  Downloads.  This  setting

controls whether  or not  Telix will  display the

free space on the disk when you start a download.

The option to turn it off is provided because the

free space display takes a  few seconds for large

hard disks.

E:              Use of Quick Dialing Bar. When  this option is on

and you select Phone / Dialing Directory or press

<Alt><D> to  access the Dialing  Directory, Telix

first  displays  the Quick  Dialing  Bar  at  the

bottom of  the screen, which  allows you  to dial

entries  without  having  to   view  the  Dialing

Directory. If you would rather go directly to the

Dialing Directory, set this option to off.

F:              Confirm Hang-Up. Setting  this option to  on will

make Telix  ask for  confirmation before  hanging

up, when Actions  / Hang-up modem is  selected or

<Alt><H> is pressed.

60   Customizing Telix

G:              Default Usage Log Setting. If  this option is set

to on, Telix will open the  default Usage Log (as

specified on the  Filenames and Paths  screen) at

program start up. Otherwise the usage log must be

manually turned on if needed.

H:              Scroll-Back Buffer  Size.  This setting  controls

the  size of  the  Telix  Scroll-Back buffer  (in

kilobytes). For changes  to this setting  to take

effect you  must  save the  changes  to disk  and

restart Telix.  The maximum is 64K.

I:              Capture File Buffer Size. This is the size of the

disk buffer that Telix should use for the Capture

File  when it  is  opened.  Values  from 0  to  4

Kilobytes are allowed. A larger buffer size means

the disk is accessed less often.

J:              Date Format. This is the format that Telix should

use when  displaying dates.  Allowed formats  are

MMDDYY, DDMMYY, and YYMMDD.

K:              Date Separator Character.  This is  the character

that Telix will  print between parts of  the date

when displaying a date.  The default character is

the hyphen (-).

L:              Time Format. This is the format that Telix should

use when  displaying times.  Allowed formats  are

12-hour and 24-hour.

M:              Time Separator Character.  This is  the character

that Telix will  print between parts of  the time

when displaying a time.  The default character is

the colon (:).

N:              Enhanced Keyboard Support. This option allows you

to  turn off  Telix's  support  for the  enhanced

(101) keyboard. This should  normally be left on,

unless there seems to be a keyboard problem.

Customizing Telix   61

O:              Shell: 16550  Buffer. This  option allows  you to

specify whether  the  buffer on  the 16550a  comm

chip should be left on or off during a DOS shell.

If it  is installed  in your  PC, Telix  normally

recognizes and enables the buffer on this chip at

all times, for greater  efficiency, but there are

still a  few  comm application  that users  might

want to run  from the shell which  can not handle

the chip with the buffer turned on.

P:              Shell: Swap to  EMS/XMS/Disk. This  option allows

you  to specify  whether  Telix  will attempt  to

remove  most  of  itself  from  the  conventional

memory  when executing  DOS  function calls.  The

default is  off for backwards  compatibility, but

most users should be able to turn this on without

problems. If therere is not enough EMS/XMS memory

available for  swapping,  Telix is  swapped to  a

temporary file named TELIX.SWP on the hard disk.

Q:              Disk Drives  installed.  Here  you may  expressly

specify  the drive  letters  of  those drives  to

appear in the  dialog box for file  uploads. This

option  is  of interest  to  users  of  removable

drives (Bernoulli, device  driven floppies etc.),

which are sometimes  not automatically recognized

by Telix. Attention: Telix will not check for the

existence of  a drive  listed here  before it  is

actually selected in the dialog box.

R:              Menu line. Here  you can choose whether  the menu

bar should appear on the  screen at program start

up. Regardless of this setting, the menu line can

be toggled  on and  off at  any time  from within

terminal  mode by  pressing  <Alt><5> or  double-

clicking with the left mouse button.

62   Customizing Telix

Modem and Dialing Settings

This screen contains settings that Telix uses to control the

modem and its output. The default configuration that Telix comes

with is set up for Hayes compatible modems. These modems use the

AT command set. By changing these settings Telix can work with

almost any modem. The parameters are:

A:              Init String.  This  string will  be  sent to  the

modem when  Telix first starts.  Different modems

might need different strings.  The default string

is set up for Hayes compatible modems. It is:

ATZ ^M ~~~ AT S7=45  S0=0  V1  X4 ^M,

where

AT     is the modem attention code,

Z      resets the modem to default state,

^M     sends  the   terminating  Carriage  Return

character to the modem,

~~~    pauses for  1.5 seconds to give  the modem

time to perform the ATZ command,

AT     is the modem attention code,

S7=45  makes  the  modem wait  45  seconds for  a

connection after dialing,

S0=0   turns off auto answer,

V1     makes the modem respond with words,

X4     makes the  modem wait for a  dial tone and

evaluate busy tones when dialing,

^M     sends  the   terminating  Carriage  Return

character to the modem.

Customizing Telix   63

To make full use of all features of your modem, you will normally

have to  modify the init string  according to the manual  of your

modem.  For most  modems  normally AT&F^M  is  a sufficient  init

string which loads the factory default settings of the modem.

B:              Dialing Prefix 1. A Dialing  prefix is what Telix

sends  to  the modem  when  dialing,  before  the

number, for  example, ATDT  for Hayes  compatible

modems and tone dialing (ATDP for pulse dialing).

There are three dialing  prefixes allowed, and in

the Dialing Directory you may choose which one to

use for each entry.  One use for this  is if your

modem supports MNP, to  define two prefixes which

turn MNP  support on and  off, thus  avoiding the

delay when you call a system that doesn't support

MNP. If  you are  dialing from  within a  private

branch exchange (PBX),  you may specify  a prefix

for internal calls and another for external calls

(the latter might for  example be AT X3 DT 0w, if

zero  is the  escape  digit and  w  is the  modem

command to wait for a dial tone).

C:              Dialing Prefix 2.

D:              Dialing Prefix 3.

E:              Dialing  Suffix. The  string  Telix  should  send

after   the  number   when   dialing.  On   Hayes

compatible modems this is  just a Carriage Return

character, ^M.

F:              Connect   String.  To   make   Telix   detect   a

connection,  you  should enter  here  the  string

which  your  modem  sends   upon  establishing  a

connection; e.g., CONNECT.

G:              No  Connect  Strings.  To  make  Telix  detect  a

connection failure, enter here up to four strings

which your modem sends when it has failed to make

a  connection  while  dialing,   for  example  NO

CARRIER, BUSY, etc.

64   Customizing Telix

H:              Hang-Up String. This  is the string  Telix should

send to the modem to make it hang up. The default

string works for Hayes compatible modems and is:

~~~+++~~~ATH0^M

where

~~~  pauses Telix for 1.5 secs,

+++  escapes the modem into command mode,

~~~  pauses Telix for 1.5 secs,

AT   is the modem attention code,

H0   tells the modem to hang up,

^M   sends   the  terminating   Carriage   Return

character to the modem.

Before sending  this string  to the modem  Telix always  tries to

hang-up the  modem by a faster  way that works with  MOST modems.

This is  done by dropping (turning  off) a special  signal called

the  DTR  (Data Terminal  Ready)  line  on  the RS-232  port.  If

dropping the DTR makes the modem  hang-up then the hang-up string

is not sent to the modem.

I:              Auto Answer String. This is the string that Telix

should send to the modem upon entering Host Mode.

This   string   should   allow   the   modem   to

automatically pick  up the  phone when  it rings,

and initialize it in any other ways that might be

necessary.

J:              Dial Cancel String.  The string Telix should send

to  the  modem  to  cancel  the  current  dialing

attempt. On most modems all  that is necessary is

a Carriage Return character, ^M.

K:              Dial Time. The  amount of time Telix  should wait

for  a  connection  while  dialing.  This  should

usually be set  to be greater than  the amount of

time the modem waits for the connection.

Customizing Telix   65

L:              Redial Pause. The number of  seconds Telix should

wait between attempts while redialing.

M:              Auto Baud Detect. If this option  is on and Telix

dials a  number  from the  Dialing Directory,  it

will check  the modem connect  message for  a bit

rate indication.  If  the bit  rate is  different

than was  specified for  the entry  being dialed,

Telix switches to  the new bit rate.  This option

only  works with  Hayes  compatible modems.  This

option must not be enabled when you have told the

modem to maintain a constant  or locked baud rate

between the  computer and the  modem, as  is done

with   nearly   all  modern   high-speed,   error

correcting modems (such as ELSA MicroLink 28.8TQ,

USRobotics Sportster, Hayes V-Series, etc.).

N:              Drop DTR to Hangup. If your  modem is not capable

of terminating a connection  by dropping DTR, you

should set this option to off, so Telix won't try

that way  and will  immediately send  the Hang-up

String (see H above) instead.

66   Customizing Telix

Filenames and Path Settings

This screen allows you to change several filename and pathname

defaults, including:

A:              Download Directory. The  disk directory  in which

Telix  should  save files  which  are  downloaded

(received). The user can  override this by giving

a full pathname when prompted.

B:              Upload Directory. The disk  directory where Telix

should look  for files to  be uploaded.  The user

can override this by giving  a full filename when

prompted.

C:              Script Directory.  The directory  in which  Telix

should  look  for script  files  when  trying  to

execute them. The user can override this by using

a full  pathname in  front of  the filename  when

specifying it.

D:              Default Startup  Script. Here  you can  specify a

script file  that will be  automatically executed

each time  Telix is  started, unless  a different

script file was specified  using the command line

switch S. Note:  The Telix title screen  will not

appear when a startup script file is executed.

E:              Default  Capture File.  The  default  name  Telix

should  use   when  opening   the  Capture   File

(normally TELIX.CAP). The user can override this.

F:              Default Usage Log. The default  name Telix should

use  when   opening  the   Usage  Log   (normally

TELIX.USE). The user can override this.

Customizing Telix   67

G:              Directory Program. The name of the disk directory

program that  Telix  should run  when you  select

Files   directory   command    (<Alt><F>)   under

Miscellaneous / DOS functions.  If  you   do  not

specify a program here Telix will use an internal

routine that also displays the  amount of time it

will take to transfer each file in the directory.

An example of a program you could specify here is

dir/p, which would  use the DOS dir  command with

pauses  at  each   page  full,  to   display  the

directory.

H:              Screen Image  File.  The file  that Telix  should

save screen images  (generated with File / Screen

Image  or  <Alt><I>)  to.  The  default  name  is

TELIX.IMG.

I:              Editor  Pathname.  The   name  of   your  editor,

including the extension of  the name (.EXE, .COM,

or   .BAT   for  batch   files),   for   example,

C:\DOS\EDIT.EXE. The  indicated file  must either

be  in  the current  directory,  in  a  directory

included  in  the DOS  Path,  or  the  name  must

include  the full  path.  You  need to  set  this

parameter only  if you  want to  run your  editor

using the File / RuN Editor or <Alt><A> command.

68   Customizing Telix

ASCII Transfers

This screen allows you to customize Telix ASCII file transfers to

your needs, so that Telix may work with almost any system.

Available settings are:

A:              Strip High Bit. If this parameter is on, the high

(most significant) bit of each character sent for

uploads, or received for  downloads, is stripped.

This can be  used to ignore parity.  On the other

hand,  the  upper  128   characters  in  the  IBM

extended ASCII character set  (128..255) are then

converted  to   the  lower  value   (0..127)  and

therefore not displayed correctly.

B:              Remote Abort  Character.  This  is the  character

which when  received  during an  ASCII upload  or

download, is  taken to mean  that the  remote end

wants to abort  the transfer. One  possible value

might be 24 (<Ctrl><X>).

C:              Local  Echo.  If this  is  on,  Telix  will  echo

characters locally  while uploading.  This should

normally be off.

D:              Expand Blank Lines. If this is  on, when Telix is

sending a file and it encounters a blank line, it

will  add a  space  to that  line.  This is  very

useful for systems that assume a blank line means

"end of file", or for uploading a message on most

bulletin boards, which often  regard a blank line

as the end of a text entry.

E:              Pace Character.  This is the  ASCII value  of the

character  that  Telix  should  wait  for  before

sending each line. If this value is 0, Telix will

not wait for any character.

F:              Line Pacing. This is the amount  of time (in 1/10

seconds), that  Telix should wait  before sending

each line. This delay is often not necessary, but

for some BBS's for example, it's vital.

Customizing Telix   69

G:              Character  Pacing. This  is  the  time delay  (in

milliseconds) that Telix should wait between each

character. As  the PC's  hardware clock  does not

have enough resolution,  this is a  software loop

based delay.  Therefore, a value  of 1  here will

delay a 4.77 MHz  XT approximately 1 millisecond,

and faster PCs for  a proportionally smaller time

period.

H:              Upload CR Translation. This is  what Telix should

do with Carriage Return characters when uploading

ASCII files. The three options are to do nothing,

to strip  them, or to  add a Line  Feed character

afterwards.

I:              Upload LF Translation. This is  what Telix should

do with Line Feed characters when uploading ASCII

files. The  three options are  to do  nothing, to

strip them,  or to add  a Carriage  Return before

each.

J:              Download CR Translation. This is exactly the same

as   setting   H  (above),   but   applies   when

downloading ASCII files.

K:              Download LF Translation. This is exactly the same

as   setting   I  (above),   but   applies   when

downloading ASCII files.

70   Customizing Telix

Protocol Options

This screen allows you to configure external protocols, as well

as a few other settings. An external protocol is simply a file

transfer protocol that is implemented outside of Telix. You

specify the name of the protocol, the key used to select it on

the file transfer menu, and define how it is to be called. You

may optionally define a string which will trigger an automatic

download much like the auto Zmodem transfer capability built into

Telix. Items A through E are the definitions for the five

external protocols allowed. When you select a definition to edit,

Telix will ask a series of questions:

Key:            The key  that should  be pressed  to select  this

protocol   when  the   file   transfer  menu   is

displayed.

Protocol Name:  The name Telix will display  on the file transfer

menu for this protocol.

Upload name:    This is the name  of the DOS batch  file or Telix

script file  to  call when  the  user selects  an

upload  using  this protocol.  If  this  filename

begins with  an @  symbol, Telix  will create  an

ASCII text file  FILELIST containing the  list of

specified or  tagged  files, one  file per  line.

Many external protocols can accept such a file as

a parameter,  and thus  limits on  the number  of

files that  can be passed  on a DOS  command line

may be circumvented.

Download name:  This is the name  of the DOS batch  file or Telix

script  file to  call  when  the user  selects  a

download using this protocol.

BAT or Script:  This controls whether the above two filenames are

DOS batch files or Telix script files.

Customizing Telix   71

DL Name:        This option controls whether or  not Telix should

ask for  the name of  the file when  performing a

download using this protocol.  In many protocols,

the sender passes the name of  the file(s) to the

receiver, so Telix shouldn't ask for the download

name.

Trigger String: This option is  a place to  enter a  string which

when  received  is indicative  of  a  file  being

received  by   the  protocol.   Telix  can   thus

automatically detect an incoming file and move to

download it  without need  of your  intervention.

This  string  must  be  consistent,  unique,  and

positively identify the occurance  of an incoming

file. Check your  external protocol documentation

to determine if  the protocol supports  a trigger

string. This option should be left blank if there

is no trigger available for the protocol.

See Appendix D (External Protocols) for

information about what information is passed to

the batch or script files when an external

transfer is selected.

Other settings on this page are:

F:              File Transfer Disk  Buffer Size. This  allows you

to set the size of the disk  buffer used by Telix

during file transfers from  1k to 10k (kilobytes)

in size.  A larger value  is more  efficient with

floppy disk  systems, while  a smaller  value can

get  around  problems with  disk  controllers  or

Extended Memory use on some hard-disk systems.

G:              Relaxed  Xmodem  Timing.   This  option  controls

whether or  not Telix  should use  relaxed timing

when performing an Xmodem  transfer. When calling

some  services, this  option  must  be turned  on

because they can not tolerate strict timing.

72   Customizing Telix

H:              Filename Guessing.  This option  controls Telix's

use of  filename  guessing. When  you selected  a

file transfer  to be  performed, telix  can often

guess the  name of  the file  to be  transferred,

based on your past keystrokes. This option may be

turned off  here. As well,  you may  specify that

Telix  gives  only guesses  which  it  is  almost

certain are filenames,  or gives its  'Best try',

which may  include guesses with  extra characters

which are not necessarily part of the filename.

I:              Zmodem Receive  Crash Recovery.  When an  aborted

Zmodem  transfer  must be  resumed,  this  option

should be turned on. While this  option is on, if

a  file  being downloaded  using  Zmodem  already

exists on the disk (and is  shorter than the file

the sender  has), Telix will  tell the  sender to

send only the bytes needed  to complete the file.

This  option  should  be   used  with  care.  The

possibility exists that a file with the same name

but different contents exists  on each system, in

which case you do not want only  some of the data

transferred.

J:              Zmodem Send Crash  Recovery. When this  option is

on, during a  Zmodem upload of files,  Telix will

try  to  tell   the  sender  to   resume  aborted

transfers. This option should  be used with care,

as some  systems do  not support  crash recovery,

and will behave unpredictably.

K:              Allow Zmodem 32  bit CRCs. Real speed  freaks may

turn off  the use  of 32  bit CRC  error checking

(still  quite  reliable  16  bit  CRCs  are  used

instead), to get on  the order of a  quarter to a

half  of  a  percent  speed  increase  in  Zmodem

transfers.

Customizing Telix   73

L:              Zmodem Window Size. Zmodem normally sends data in

streaming mode,  never pausing  unless the  other

side requests  a re-send  due to  an error  being

detected.  In  some  environments,  a  pause  for

acknowledgments can be more efficient. The window

size (in kilobytes) is the maximum amount of data

the protocol  should  send before  waiting for  a

reply.  A value  of  0  indicates streaming  mode

should be used.

M:              Zmodem  File Type.  This  option  allows  you  to

specify whether  the file being  transferred with

the Zmodem protocol is  Binary, ASCII, or Either.

Binary means that Telix will make sure no end-of-

line  conversion is  done  on  files received  or

sent. ASCII means that on a  download, as long as

the other side doesn't  override this, Telix will

assume the file being received is a text file and

will  make  sure  the end  of  each  line  has  a

Carriage  Return  followed by  a  Line  Feed,  by

adding the CR if the file has only LFs at the end

of each line. An ASCII  setting when sending will

make Telix  tell the other  system to  do end-of-

line conversion,  although the  other system  may

override this. Finally, a  setting of Either (the

default),  will make  Telix  assume  the file  is

binary,  unless   the   other  system   indicates

otherwise. Do  not  use a  setting  of ASCII  for

files that  are not ASCII  text files,  as binary

files will be corrupted by this option.

74   Customizing Telix

Kermit Transfers

This screen allows you to configure Telix's implementation of the

Kermit protocol to your needs. The options will not be described

here, due to the fact that the Kermit protocol is relatively

complicated, and if you must change one of these parameters, you

should generally already know what it means.

Comm Port Setup

This screen allows you to configure/define the communications

ports for the standard communications mode, i.e. the direct use

of a serial interface. In INT14 mode, these settings have no

meaning! Telix supports definitions for 8 comm ports. By default,

the first four comm ports are defined to the standards for COM1

to COM4. The last four comm ports are by default duplicates of

COM1. For each port you may define the base address (in

hexadecimal) of that port, as well as the interrupt request

number (IRQ) to use. Unless you are absolutely sure about your

information, do not modify these values! Wrong values could cause

unexpected and possibly disruptive results.

INT14/FOSSIL Mode

This option allows you to set Telix to the INT14 mode, where data

are not sent directly over a serial interface, but via the BIOS

interrupt 14h, which is used for example by modem sharing

programs in networks and by FOSSIL drivers (e.g. for passive ISDN

adapter boards). A suitable program of this type must be loaded

before Telix is started. If Telix does not detect a suitable

driver software, it will report this and switch back to the

standard communications mode. If a driver is found, Telix

activates the INT14 mode and attempts to initialize the driver.

If this fails, Telix will ask you to select a different INT14

communications port and retry to initialize the driver. If you

select "none" or press <Esc>, Telix returns to the standard

communications mode.

Customizing Telix   75

Language Setup

This menu option allows you to select the language for the Telix

user interface, i.e. for the menu commands and dialog texts.

Telix presents you a menu with all available languages, i.e.

those languages for which a language file with the extension .LNG

(e.g. TELIXGB.LNG for British English) is found in the Telix

startup directory (the directory where Telix looks for the

configuration file TELIX.CNF).

NOTE            When the  language is  changed, certain  shortcut

keys for  Telix  commands might  change as  well,

since they may be differently defined to meet the

different national keyboard layouts. For example,

the  German  version  uses  <Alt><N>  instead  of

<Alt><9> for the script recording function.

Saving Changes and Leaving

Any modified settings can be saved to the configuration file (so

that they will be remembered when you next use Telix) by

selecting the Write setup to disk option. If you want to keep any

changes just for the current session, then exit the Preferences

Menu using the Exit option.

Terminal Emulation   77

Terminal Emulation

A video display terminal is basically a relatively dumb computer

which is linked up to another computer (over a cable or modem

link) and displays what that computer sends to it. A mainframe or

mini computer has a large amount of processing power and can

drive a number of terminals at the same time.

There are many different terminal types. Each one follows its own

codes for controlling how information is displayed on the screen.

For example, there are sequences to clear the screen, move the

cursor, scroll the display, and so on. Most terminals also have

special keys which send special sequences back to the main

computer.

Telix can emulate several terminals (i.e. react to certain

control sequences in the same way as the respective terminal).

While in Terminal Mode, you may change the terminal type Telix is

currently emulating by selecting

Configuration / Terminal Emulation or pressing <Alt><T>. The

default terminal can also be set in the Terminal Options page

under Configuration / Preferences. Following are the choices

Telix offers:

TTY

A TTY (Teletype) terminal is very stupid. It basically displays

all the values it receives, except for about 5 or 6 Control codes

(such as Carriage Return), which move the cursor and ring the

bell. Telix does not have any special key definitions for this

terminal type.

ANSI-BBS

This is a subset of the real ANSI terminal type defined by the

American National Standards Institute. Many BBS programs, when

told to do so, can send sequences supported by this terminal to

make screen display more interesting and colorful. This emulation

supports some known bugs in the DOS ANSI.SYS driver that are not

part of the ANSI standard per se.

78   Terminal Emulation

The ANSI.KEY file contains definitions for the arrow keys, and is

automatically read when this terminal is selected. These

definitions are supported by some full screen editors, for

example.

ANSI

This is the real ANSI terminal type defined by the American

National Standards Institute. Many BBS programs, when told to do

so, can send sequences supported by this terminal to make screen

display more interesting and colorful. This conforms to the ANSI

committee definitions of ANSI.

VT102

The DEC VT102 terminal is one of the most popular (and most

emulated) terminals in existence. It was developed by Digital

Equipment Corporation. The VT102 terminal has a large number of

functions. Telix supports all of these except for 132 column mode

and double height mode. As well, Telix only emulates double width

mode, by adding a space after every character.

Telix fully supports both normal and application cursor mode in

VT emulation. The VT102 keyboard layout used by Telix is stored

in the VT102.KEY file, which is automatically read by Telix when

VT102 terminal emulation is selected. Once Telix has loaded this

into memory, the definitions can be edited through the use of the

Configuration / Key defs./macros or <Alt><K> function.

The default keyboard layout is best suited for the (newer) 101-

key style keyboards with separate arrow and numeric keypads, and

functions keys across the top. In this layout, press <F1> to <F4>

to simulate the VT's <PF1> to <PF4>. With the <NumLock> key down,

the PC's numeric keypad will simulate the VT's numeric keypad,

while the arrow keys on the PC's numeric keypad (with the

<NumLock> up) or the separate arrow keypad (at all times) will

simulate the VT's arrow keys. Note that Telix will only recognize

the separate arrow keypad if you turn on the Enhanced Keyboard

setting on the General Options screen of the Configuration Menu.

Terminal Emulation   79

Another keyboard layout is available that is possibly better

suited to older style keyboards with function keys on on the left

side and no separate arrow keypad and numeric keypad. To make use

of this alternate layout, while at the DOS prompt type:

ren VT102.KEY VT102OLD.KEY

ren VT102ALT.KEY VT102.KEY

This layout is similar to one used by many comm programs, in

which the normal function keys

(<F1> - <F10>) represent the left side of the VT's numeric

keypad, while the shifted function keys (<Shift><F1> -

<Shift><F10>) represent the right side. The arrow keys are also

defined to send the proper sequences.

VT52

The DEC VT52 is another relatively common terminal type, that is

considerably simpler than the VT102. Telix's key definitions for

the VT52 are stored in the VT52.KEY file.

AVATAR

AVATAR is another terminal emulation, which assigns escape

sequences to some of the keys of the cursor keypad. The key

definitions are stored in the AVATAR.KEY file. Note that Telix

only supports AVATAR Level 0.

Using Script Files   81

Using Script Files

Available to supplement Telix is a pair of incredibly powerful

'script' or command languages, called SALT (Script Application

Language for Telix) and SIMPLE (SALT Implementation). In the

simplest terms, a script file is a sequence of commands for Telix

to follow, written using a certain format. Script files can do

many useful things like automatically logging on to a host,

setting up special parameters, performing unattended file

transfers, and many other tasks. A script file can be linked to a

Dialing Directory entry so that it is run automatically when that

entry is connected to, and supplies the needed logon information.

Like any real programming language, SALT has a large number of

features. Because of this, programming in SALT is not easy for

the beginner who has no knowledge of any programming concepts.

However, SALT scripts can be used to different degrees. The

advanced user can program complex applications, while even a

novice can modify the sample scripts included with Telix, and run

scripts supplied by others.

For advanced users, a separate manual describes SALT and its

syntax. The rest of this section describes how to create, run and

compile the various forms of scripts.

Creating and Compiling Scripts

SALT scripts are created as text files with any ASCII text editor

(such as the DOS editor) and are normally saved with the

extension .SLT. You can edit a script directly from within Telix

by selecting the Script / Edit command or pressing <Shift><F1>.

Telix will ask you for the filename of the script to be edited

and automatically start the editor program specified under

Configuration / Preferences / Filenames and Paths. If no filename

extension is given, Telix will assume .SLT.

82   Using Script Files

Before a SALT script can be used, it must be 'compiled'. The

CS.EXE program included with Telix takes the ASCII 'source'

scripts that the user writes, and compiles or converts them to a

form that is easier for Telix to process, usually takes less

space, and loads more quickly. You can call the SALT compiler at

the DOS prompt by entering CS, followed by a blank and the

filename of the script to be compiled. However, you can also

compile a script directly from within Telix by selecting

Script / Compile or pressing <Ctrl><F9> and entering the name of

the script file to be compiled (for this function, CS.EXE must be

present in the Telix directory or in the default script

directory). In both methods Telix will assume .SLT if no

extension is given. CS will scan through the source file and

produce the compiled version, using the same base name but the

extension .SLC. If a syntax error is discovered while compiling

the script file, CS will report it and abort. In that case the

error should be fixed and CS run again.

Remember, every time you make a change to the source file, you

must re-compile it, otherwise Telix will still run the old

compiled version. This sounds tedious, but in practice once a

script is developed it doesn't change very often.

NOTE            For Telix 3.50 or newer, version 1.51 or newer of

the SALT compiler CS.EXE should be used.

Using Script Files   83

Executing a Script

If you have an already compiled SALT script (ending with the

extension .SLC (for example, the QDHOST.SLC Host Mode script

file), it is very easy to execute. While in Terminal Mode, press

<Alt><G> or select Script / eXecute. Telix will ask for the name

of the script file to run. If you supply no extension, .SLC is

assumed. If a special directory where script files are to be

found has been defined, Telix will look there for script files,

unless a path is given. This script directory can be defined

under Configuration / Preferences / Filenames and Paths. Telix

will then load the script file and follow its instructions. To

abort a script while it is executing, press the <Esc> key or

select Script / Abort (however, the abort function may be

disabled in the script itself). Sometimes you may have to press

<Esc> twice, the first time to abort the current function, and

the second time to abort the script itself.

A script can also be linked to an entry of the Dialing Directory,

so it will be executed automatically each time a connection to

this entry has been reached. Furthermore, a startup script can be

defined, which is executed automatically each time Telix is

started.

SIMPLE

For those users not comfortable in a structured programming

environment, Telix provides a SALT Implementation, or SIMPLE

scripting language. SIMPLE is a non-structured language that is

converted to the more complex SALT for the user prior to actually

compiling it. SIMPLE scripts need to be compiled with the CSS.EXE

program. To compile a SIMPLE script at the DOS prompt, type CSS,

followed by a space and the name of the SIMPLE script file you

want to compile. The extension .SIM is assumed if none is given.

84   Using Script Files

If you use the Script / Compile command (or <Ctrl><F9>) from

within Telix instead and enter a filename with the extension

.SIM, CSS is called automatically (for this function, CSS.EXE

must be present in the Telix directory or in the default script

directory). In this case, it is important to expressly supply the

extension .SIM, since otherwise Telix would look for a .SLT file

and attempt to call the SALT compiler CS instead. CSS will scan

through the source file and produce a compiled .SLC file. If a

syntax error is discovered while compiling the script, CSS will

report it and abort.

SIMPLE scripting is described in detail in the Scripting

Reference guide.

Learning a Script

Some operations you will perform in Telix are highly repetitive,

such as logging onto an online service. The user name, password,

and other information is always entered in the same order, the

same way, every time you call. You can use the Script Learn

function to create an automatic log-on script to do this for you

each call (the script should be linked to the respective Dialing

Directory entry).

To learn a sequence of responses to prompts, toggle the learn

mode on from Telix Terminal Mode by pressing <ALT><9> or

selecting Script / Learn. Enter the name of the script to learn

when prompted. .SLT is assumed to be the filename extension if

none is given.

Telix will record your responses to the various prompts of the

online service until learning mode is turned off with <ALT><9>

(or Script / Learn) again. When turned off, Telix saves the

learned script. Remember, the learned script must be compiled

prior to use.

It is important while learning a script to remember to wait for

the complete prompt to be displayed before responding.

Unpredictable results can be exhibited if a fast typist or a user

familiar with a service types ahead of the prompts.

Using Script Files   85

Learned scripts, while useful, are not always completely

accurate, and may not work without editing. If the prompt being

responded to contains information that can vary (such as the

number of minutes left online), then the .SLT file may need to be

edited prior to being compiled. An example is the following

prompt on a BBS, which might read:

(4 used, 86 left) Main Board Command?

If there are not exactly 86 minutes remaining on your next call,

a learned script containing this prompt will not function

properly. The line in the learned script:

waitfor("(4 used, 86 left) Main Board Command?", 30);

should thus be edited to read:

waitfor("Main Board Command?", 30);

for proper operation of the learned script.

Host Mode                                                      87

Host Mode

Telix allows you to set up your computer as a remote host, so

that others may call your system and perform operations like

transferring files and chatting. In short it is like having a

small BBS on your system, without all the setup work. Telix

provides two such host modes, both implemented in Telix's

powerful SALT script language. QDHost is a 'Quick and Dirty' Host

Mode containing the basic features of a remote host, including

password security and remote shells to DOS. If you have purchased

the SALT compiler, a more powerful host, HOSTPLUS, is also

provided, offering the functionality of a full-featured BBS with

multiple message bases and file areas, multiple security levels

and individual password security and user levels. The operation

of HOSTPLUS is described in the text file HOSTPLUS.DOC.

There are a few requirements to running Host Mode. Your modem

must be able to pick up the phone when it rings. Your modem must

not be set to override (always keep on) the Carrier Detect signal

it supplies, so that Telix can tell when it is connected to a

caller. Telix must also be able to hang up the modem by dropping

(turning off) the DTR line on the RS-232 port. If one of these

requirements is not met you probably cannot run QDHost.

To run the QDHost script, select Script / eXecute or press

<Alt><G> while in Terminal Mode, and then enter QDHOST as the

script file name. The script QDHOST.SLC will then be run. Several

Host Mode configuration settings are kept in a file called

QDHOST.CNF. The first time you run the Host Mode script, it will

not find this file, and automatically run the Host Mode

configuration script QDCONFIG.SLC, which will create the needed

file and allow you to change the default values. You should run

QDCONFIG should you need to configure the Host Mode at any time

in the future. QDCONFIG will present you with the following

parameters:

A:              Level  One Access  Password.  The  password  that

callers must successfully enter to be admitted to

the Host  Mode and have  Level One  access. Level

one access  allows  only files  contained in  the

specified  QDCONFIG  download   directory  to  be

downloaded.

88    Host Mode

B:              Level Two Access  Password. This is  the password

that  callers  must  successfully   enter  to  be

admitted to  QDHost  and have  Level Two  (Sysop)

access. Level  two  access allows  the caller  to

download any  file on  the system  by giving  the

path.

C:              DOS Shell  Password.  This is  the password  that

users of QDHost  have to enter to  use the Remote

DOS Shell option.

D:              Shut  Down  Host  Mode  Password.   This  is  the

password that  callers  must enter  to shut  down

QDHost, when they press <Ctrl><Z>.

E:              Host Download Directory.  Level 1  access callers

will only  be able to  download (have  Telix send

them) files that are in  this directory. As well,

the  Files  command  will   show  them  only  the

contents  of this  directory.  Do  not under  any

circumstances define  this directory to  the same

name as  the directory you  run Telix in.  If you

did  that,  callers  could  download  the  QDHost

configuration file,  which is  stored there,  and

see your  passwords. Level  2 access  callers may

specify  any  download  path,  but  this  is  the

default.

F:              Host Upload  Directory.  Level  1 access  callers

will only be able to upload (send to Telix) files

into this directory.  Level 2 access  callers can

specify  any path,  but  files  will go  here  by

default.

G:              Connection  Type. This  can  be  either Modem  or

Direct. Use  Modem if your  host system  is being

accessed  through  a  modem,  or  Direct  if  two

computers are  hard-wired. This stops  Telix from

trying  to initialize  a  non-existent modem.  or

trying to determine the caller's baud.

Host Mode                                                      89

H:              Locked Modem Rate.  For error  correcting modems,

QDHost needs to  know if the DTE  rate (the speed

Telix communicates  with your modem)  is constant

from call to call, no matter  what speed the user

calls in at. Set this value to the fixed DTE rate

(usually 9600 for  2400MNP modems, and  19,200 to

57,600 for 9600+  baud modems). If your  modem is

not error correcting  or you are unsure  what the

value should be, set this value to zero.

There is one other important parameter which must be properly set

for QDHost operation. This is the Auto Answer string, which is

defined under Configuration / Preferences / Modem and dialing.

This string is sent to the modem when Telix enters QDHost, and

should make the modem able to pick up the phone when it rings.

The default string is set up for Hayes compatible modems. It is:

AT S0=1 ^M

AT   is the modem attention code,

S0=1 tells the modem to answer after one ring,

^M   sends the terminating Carriage Return character.

NOTE            If you  want to  disable the  automatic answering

after the host mode is terminated, insert S0=0 in

the    modem   init    string    defined    under

Configuration / Preferences / Modem and dialing.

There are two optional text files which are displayed to callers

if they exist, called LOGO.MSG, and WELCOME.MSG. They are

described below.

90    Host Mode

Once QDHost has been configured, it can actually be run. Telix

will make a note in the QDHost audit file QDHOST.LOG and then

will immediately send the Host Mode init string to the modem.

Telix then waits for a caller. While in QDHost on the local side,

the user can always press <Esc> to exit Host Mode, or <End> to

terminate the current caller. He can also enter commands for the

remote caller, so that a novice caller can be helped along by the

host operator. Also, at any time, the current caller will be

automatically terminated if there is inactivity for five minutes,

or the connection is lost. This is all noted in the host audit

file. If the caller has only access level one, references to

other disk directories than the ones defined in QDCONFIG are

ignored, and all operations take place in those directories. As

well, Telix protects your files from being accidentally

overwritten by the caller.

When the Carrier Detect signal turns on Telix knows a caller is

online. At this point Telix will try to determine the caller's

baud rate. Most Hayes compatible modems can be initialized to

send the string CONNECT baud where baud is the baud rate of the

caller, when they sense a connection. If Telix sees this string

it will switch to the appropriate baud. If this string is not

received within a few seconds of the connection, Telix will leave

the baud rate at whatever it was. Also, it should be mentioned

that Telix can not determine the caller's other settings like

parity, data bits, and stop bits. These must already be matching

Telix's settings.

At this point, if the file LOGO.MSG exists in the Telix

directory, it is displayed to the caller. It should basically

identify the system. Then Telix will ask the caller for his/her

name. The name must be at least five letters long and is only

used when indicating the caller in the Host Mode audit file. If a

password has been defined Telix will ask the caller for it. The

caller has three tries to complete the password. All password

attempts, successful or failed, are written to the Host audit

file. If the caller enters the Level One access password, he may

manipulate only the disk directories defined in QDCONFIG. Any

other directories can not be accessed. If he enters the Level Two

access password, the caller may use full pathnames in all file

specifications. The level 2 password should only be kept for

yourself or those you completely trust, as a Level 2 caller can

access any files on your system.

Host Mode                                                      91

If there is a file called WELCOME.MSG in the Telix directory, it

is then shown to the caller. This file can contain further

identification or news, that you only want valid callers to see.

After each 23 lines the caller is prompted to press any key to

continue. As well, the caller may press <Ctrl><S> to pause at any

time, and <Ctrl><C> or <Ctrl><K> to abort the listing.

After the WELCOME.MSG file is displayed the caller is presented

with a menu of options:

Files    Type    Upload    Download    Shell    Chat    Goodbye

The action to be taken is selected by entering its first letter.

Case does not matter. As mentioned already, the host operator is

free to enter any keystrokes for the caller.

Help            The Help option  prompts the caller to  enter the

first letter of a menu option and then displays a

help  text   describing  the   function  of   the

respective option.

Files           The Files  option allows the  caller to  list the

files in the  current disk directory.  The caller

must press a key after each screen. The output is

not echoed on the local screen. If the caller has

access Level 2 he/she is prompted for a filespec,

which may include the * and ? wildcard characters

(see your  DOS manual), so  that the  contents of

other   directories   than  the   Host   Download

Directory may be listed.

Type            The Type  option allows  the caller  to view  any

ASCII file in the Host  Download Directory, or in

any directory  for  access Level  2 callers.  The

same  format and  commands  are available  during

this  command  as  for   the  displaying  of  the

optional message file WELCOME.MSG.

92    Host Mode

Upload          The Upload  option allows  the caller  to send  a

file  to  the  host.  The  caller  is  shown  the

following menu  (here slightly reformatted  so it

could fit):

Modem7    SEAlink    Xmodem    1k-Xmodem    G-1k-Xmodem    Ymodem

YmodEm-g   Zmodem

Which protocol?

and should select the appropriate protocol by its

first letter (or <E> for Ymodem-g). If

appropriate the caller is also asked for the

filename. The transfer is then initiated. Level 1

callers may only upload files into the Host

Upload Directory defined with QDCONFIG.

Download        The Download command allows a caller to receive a

file from  the host. The  caller must  select the

protocol as described  above, and then  must tell

Telix the file(s)  to send. The transfer  is then

initiated.  Level  1  access   callers  may  only

download files  from the Host  Download Directory

defined with QDCONFIG.

Shell           The Shell  command is  a very  powerful but  also

very dangerous command.  It allows the  caller to

run a DOS  shell on your system,  except that the

caller receives the output, and the caller enters

the keystrokes. This command is basically similar

to entering  CTTY  COMx at  the  DOS prompt.  The

remote side has complete  control of your system.

This is obviously  very dangerous, as  the caller

can format  disks, delete  files, or  perform any

other operation. To provide  security, a password

must be entered  to use the remote  DOS shell. Be

very careful of  whom you give this  password to,

as  they can  do  anything  they want  with  your

system. As  well, don't leave  it at  the default

that Telix comes with.

Host Mode                                                      93

There are a  few limitations in the remote DOS  shell. The caller

will see  program output  only if the  programs use  standard DOS

output. Programs  that write  directly to  the video  screen will

work,  but will  not  be  seen by  the  remote  caller. As  well,

programs that use non-DOS methods of  getting keystrokes will not

receive the caller's keystrokes. Finally,  under some systems, if

the caller presses  Backspace at the DOS prompt  when the current

line is  empty, DOS will hang  on the Host machine.  As these are

functions of DOS,  there is nothing that can be  done about these

limitations. The  remote shell also  only supports COM1  and COM2

when using CTTY.

A potentially powerful feature of the shell function is the

ability to override the use of CTTY and use your own redirector,

such as Marshall Dudley's DOORWAY.EXE. Do not confuse DOORWAY.EXE

with Telix's DOORWAY mode. If the file RSHELL.BAT exists in the

same directory that QDHOST.SLC is in, QDHost will run that batch

file instead of using CTTY, and return to QDHost when it is

complete. DOORWAY.EXE is an excellent alternative to CTTY when

both users need to see the screen, or when programs that write

directly to the screen (such as Word Perfect) must be run from

remote. The following is a sample RSHELL.BAT for use with the

Doorway program:

DOORWAY com1 /M:100 /O: /V:D^U /C:DOS

Note that  the com  port must  specified by  number in  the batch

file. However,  CTTY only supports  COM1 and COM2,  while Doorway

can  support up  to COM4.  If you  receive the  message that  you

cannot shell due  to DOS limitations, DOORWAY.EXE can  be used as

an alternative.

Chat            The Chat command  allows the caller to  chat with

the host  operator. When  the caller  presses <C>

the host operator is paged  for 20 seconds. Press

the Space  bar to  chat with  the caller,  or any

other key not  to. Accepting the chat  will place

you in Chat Mode, exactly as if  you had used the

<Alt><Y>  command.  The caller's  keystrokes  are

echoed back  to him/her  however, so  that he/she

can see them.

94    Host Mode

Goodbye         Finally, the Goodbye command allows the caller to

log off  the host. An  entry noting the  time and

date is  made in the  host audit file,  and Telix

then waits for a new caller.

The caller may also use one other command that is not displayed

in the menu. This is to shut down QDHost Mode. If the caller

presses <Ctrl><Z> and successfully enters the shut down Host

password defined with QDCONFIG, the QDHost Mode is shut down and

Telix returns to whatever it was doing before QDHost was

initiated.

Appendices   95

Appendices

A    Keyboard Command Summary

<F1>            Display Help/Status Screen

<Ctrl><F1>      List Command Line Options

<Alt>           Activate Menu Bar

<Alt><A>        Run Editor

<Alt><B>        Scroll Back

<Alt><C>        Clear Screen

<Alt><D>        Dialing Directory

<Alt><E>        Local Echo on/off

<Alt><F>        DOS and File Functions

<Alt><G>        Run Script File (Go)

<Alt><H>        Hang-up Modem

<Alt><I>        Screen Image

<Alt><J>        Jump to DOS Shell

<Alt><K>        Keyboard Definitions

<Alt><L>        Capture File

<Alt><M>        Misc. Options

<Alt><O>        Preferences Menu

<Alt><P>        Comm Parameters

<Alt><Q>        Queue Redial Last Numbers

<Alt><R>        Download (Receive) Files

<Alt><S>        Upload (Send) Files

<Alt><T>        Terminal Emulation

<Alt><U>        Usage Log

<Alt><V>        DOS Command

<Alt><W>        Translate Table

<Alt><Y>        Chat Mode

<Alt><X>        Exit Telix

<Alt><5>        Toggle Menu Bar on/off

<Alt><8>        Toggle Status Line on/off

<Alt><9>        Learn a Script

<Alt><F9>       Edit a Script

<Ctrl><F9>      Compile a Script

<Alt><=>        Toggle DOORWAY Mode on/off

<Ctrl><@>       Printer Log on/off

<Ctrl><End>     Send Break Signal

<Shift><Tab>    Add Line-feeds on/off

<PgUp>          Upload (Send) Files

<PgDn>          Download (Receive) Files

Appendices   97

B    Troubleshooting

Problem:        Telix and  the modem do  not seem  to be  able to

detect busy signals.

Solution:       Some modems (especially older 1200  bps units) do

not have the  capability to detect  busy signals.

Assuming yours  does, you'll still  probably have

to edit  the default  modem Init  String. The  X1

that some modems  use by default does  not enable

busy detection in  most modems. Use a  value like

X3, X4, or higher in the init string.

Problem:        When the QDHost  mode script is run  Telix always

thinks that  a caller  is online  and immediately

asks for the  caller's name or Telix  always says

that a Hang-up operation failed  even when it did

in fact successfully hang-up the modem.

Solution:       Your modem  is  almost  certainly overriding  the

true state of the Carrier  Detect signal. This is

the factory default on most modems, but should be

disabled. For  proper operation,  Telix needs  to

see  this signal  on  when  connected to  another

computer, and off when not. If your modem has dip

switches, as  do most 1200  bps units and  all US

Robotics  external  Couriers,   switch  number  6

usually  controls this  and  must  be in  the  up

position. If your modem does not seem to have any

dip switches (look carefully, sometimes the front

needs  to   be  popped   off),  it   is  probably

controlled solely  by software  commands, as  are

most  2400  bps  or  faster  units.  Just  a  few

examples  of  these  are   the  Hayes  2400,  ATI

2400etc., GVC  2400, and  many others.  For these

modems,  adding  &C1 in  the  modem  Init  String

(before the final ^M (Carriage  Return) is a good

spot) will configure the modem properly.

98   Appendices

Problem:        When trying to use a multi-tasking system like MS

Windows, Topview, or DoubleDOS, with Telix in the

background, window displays bleed  through to the

active partition.

Solution:       Under  Configuration / Preferences,   select  the

Screen and colors settings option, then select as

the  Screen  Write  mode,  BIOS  calls  used  for

writes. Screen updating  will be slower  but will

not bleed through.

Problem:        When redialing Telix does not seem to know when a

connection has been reached.

Solution:       Telix knows when a connection has been reached in

one  of two  ways:  when  it receives  a  Connect

string  from  your modem,  or  when  the  Carrier

Detect signal turns on (if it was off). Make sure

that the  Connect string  is properly  defined in

the Configuration  Menu, or  check if  your modem

does turn on the Carrier Detect signal regardless

of whether or not there is a connection.

Problem:        Telix doesn't work with a certain modem.

Solution:       Telix is  set by  default to  use the  Hayes 'AT'

modem command standard. There are modems that are

not  Hayes  compatible  however,  and  use  other

commands  to dial,  hang  up,  and perform  other

tasks. Make sure that if your  modem is not Hayes

compatible Telix has been  properly configured to

its commands.

Appendices   99

Problem:        When trying  to  use the  DOS  Shell, or  another

function  that  uses DOS,  Telix  warns  that  it

cannot find a file called COMMAND.COM, and aborts

the function.

Solution:       The  file   COMMAND.COM   is   the  DOS   command

interpreter. Telix must be able to find it to use

many DOS  functions. The location  of COMMAND.COM

is stored  in an environment  variable (explained

in your  DOS manual)  called COMSPEC.  COMSPEC is

set at boot-up, but  if you boot of  a floppy and

then change to another floppy or  a hard disk, it

will not  point to  the right  place anymore.  In

short, make  sure that  COMSPEC always  points to

the location of COMMAND.COM,  or that COMMAND.COM

is in the current directory.

Problem:        When calling some systems,  especially large ones

such as  CompuServe or  The Source,  all incoming

characters look like garbage.

Solution:       The communications parameters are probably wrong.

Most of  these  systems need  a  setting of  Even

parity, 7  data  bits, and  1 stop  bit. This  is

different from  the normal  standard of  N81 used

for most bulletin boards.

100  Appendices

Problem:        During transfers  with a  high speed  modem, many

CRC and/or timeout errors occur.

Solution:       First ensure that  CTS/RTS hardware  flow control

is enabled and that DSR/DTR hardware flow control

is disabled both in Telix in the Terminal Options

section of  the Configuration /  Preferences menu

and in your modem (refer to your modem manual for

instructions on  setting up your  modem properly,

or  use the  supplied  MODEMCFG.EXE program).  If

this   fails,   it   may   simply   be   hardware

limitations. Sometimes such  hardware limitations

can be  circumvented by  setting Drop  RTS during

disk writes  in the  Terminal Options  section of

the Configuration / Preferences menu.

Many high-speed  modems, especially  in a  multi-

tasking   (Windows,  DESQview,   TopView,   etc.)

environment or on XT  or slower AT-class machines

are simply  too fast  for the  hardware, and  may

need some help to prevent lost characters. A UART

(Universal Asynchronous  Receiver-Transmitter) is

a chip  found on  every serial  card or  internal

modem. Most serial cards  or internal modems come

stock with 8250 or 16450 chips that are not rated

for high speed modems.  A replacement chip called

the 16550 will likely eliminate such problems.

Problem:        When I run Telix from my menu program it tells me

Unable to find/open ANSI.KEY and goes back to the

menu program.

Solution:       ANSI.KEY is a file required  for Telix operation,

but due  to the  menu not  changing to  the Telix

directory,  Telix cannot  find  this file.  Telix

expects to  find all of  its system files  in the

current directory or in  the directory pointed to

by the  TELIX environment  variable.   By placing

the command:

SET TELIX=C:\TELIX

in  your  AUTOEXEC.BAT (modified  for  your  own Telix  path,  of

course). Telix will then know to  look there for all of its files

if they  are not  in the  current directory.  There should  be no

spaces in the command as above, other than between SET and TELIX.

Appendices  101

Problem:        I have  a  new 28,800  or 14,400  bps modem,  but

Telix doesn't support 28,800  and 14,400 as speed

options.

Solution:       This is  one  of the  great misconceptions  about

high  speed  modems,  so   you're  not  alone  in

wondering  this. Let  us  try  to detail  why  it

doesn't matter, and  at the same time  give you a

bit of an idea what's going  on behind the scenes

when you call another modem...

The link  to get from your  computer to the other  computer looks

much like this:

Your Computer <--> Your modem <--------> Their modem <--> Their computer

DTE rate           DCE rate            DTE rate

38,400             14,400              57,600

As you can see, it is really a series of three links; one between

your computer and your modem, one between the two modems, and one

between their modem  and their computer. What  might surprise you

is  that  each of  these  three  rates  can  be, and  often  are,

completely different, as above. So you  know, DCE stands for Data

Communications Equipment (i.e. a modem to  modem link) and DTE is

Data Terminal  Equipment (i.e. terminal  to modem link).  You are

not concerned with  the final link, the remote DTE  rate. That is

up to the  remote site, and does  not matter at all  to you. Once

the data leaves your modem, and  is received by theirs, it is out

of your hands.

102  Appendices

Your modem  likely has  either MNP5  or V.42bis  data compression

built in.  For transferring uncompressed files,  these modems can

be extremely efficient in compressing the  data before sending it

- sometimes as  much as 4 times compression (25%  of the original

size).  Even  a  simple error  correction  (MNP4,  V.42)  already

reduces the actual  amount of data to be  transferred by removing

the start  and stop  bits of  each character  before it  is sent,

since they are not needed in error-corrected connections.

If the modems can take 1000  characters from Telix, and then turn

it into  perhaps as  little as  250 characters  with compression,

your modem still  transmits, for example at 14,400  bps and would

need 1000 characters from the comm program to transmit a mere 250

characters. In order to keep the  DCE link flowing with data non-

stop, Telix has to  send data to your modem at  4 times the speed

the modem is talking to the  other modem (in the best case, which

almost never happens).  Thus, the DTE (Telix to  modem rate) must

be higher than the DCE (modem to modem rate) by a good margin, or

the modems will sit idle frequently, waiting for the comm program

to supply it  with enough data. Since you have  no way of knowing

how much the  data will be compressed, or at  what speeds the two

modems will actually  connect up at, you should  ALWAYS leave the

DTE rate  on your end (the  link between Telix and  your modem as

specified in  the Telix  configuration) locked  in, or  fixed, at

that  high rate  that can  accommodate the  most efficient  case,

since that most efficient case can  occur at any time. As long as

it does  not occur, the data  flow control (CTS/RTS  or XON/XOFF)

saves  your modem  from being  flooded  with more  data from  the

computer than it can buffer.

That's why you're always advised by  MODEMCFG.EXE to set the comm

program's speed,  as well  as all  Dialing Directory  entries (no

matter how  fast the board actually  is), to a speed  higher than

the 9,600 or 14,400 you really have. Typically, you'll be told to

use 19,200 or  38,400 (nowadays, typically 38,400,  and even some

will say  57,600 or  115,200). But the  important thing  is, that

speed is constant. Your DTE (program  to modem rate) always stays

the  same, so  that when  that most  efficient case  comes along,

you're  ready. Remember  that you  might  need a  fast UART  (see

above, typically for rates greater than 19,200).

Appendices  103

Problem:        When trying to  transfer a file, Telix  just sits

there  saying  Waiting  to  send  or  Waiting  to

receive but nothing ever happens.

Solution:       When a user  is downloading, the other  system is

by definition uploading to him. Both systems must

know exactly  what  is happening  at every  given

moment,  and  this  is  especially  true  at  the

beginning of the transfer.

First the downloader must  tell the remote system

(the  one  to be  downloaded  from)  that  he/she

requests a  download.  On most  systems, this  is

accomplished with a "Download" command.

The sending system  will then ask  the downloader

to choose a protocol. You may choose any one that

Telix supports, but we recommend  Zmodem if it is

available,  and   1K-Xmodem  (sometimes   labeled

Ymodem) if Zmodem is not  available. In any case,

the important thing to remember  is that both the

sender and  the receiver must  be using  the same

protocol, and it must be agreed upon in advance.

Perhaps before choosing  a protocol, you  will be

asked what files  you wish to download.  Then the

system may tell you that it is  ready to send the

files.  If you  have  selected  Zmodem, and  have

Zmodem autodownloads  on in  Telix (the  default)

you should  not have to  do anything  more. Telix

will sense the Zmodem transfer coming and go into

Zmodem receive  mode. Sometimes this  will appear

as  "garbage"  like  an  up  arrow,  a  bunch  of

asterisks, and numbers like 0's  and 8's. This is

a signal to start!

104  Appendices

The  most   important  thing  to   remember  when

downloading is  that first you  have to  tell the

other system what to send and how to send it, and

let it get  started. As soon as  the other system

starts, you generally have about 30 to 60 seconds

to start your receive with  the same protocol. It

is crucial that  both sides know that  a transfer

is taking place. You cannot start yours early, or

the other side will never send the file.

Thus, don't  hit <Alt><R>  (or <PgDn>)  until you

are sure  the other  side is  ready to  send, and

ready for you  to tell it  that you are  ready to

receive (<ALT><R> does this automatically).

Problem:        When trying to compile a script I get the message

"Unable to  open  file" even  though  I know  the

script is present.

Solution:       Some OEM versions of DOS 2.11 (notably, the Tandy

DOS burned  into  the 1000  HX) are  incompatible

with the compiler used in  these cases. This does

not  apply   to  Telix   itself.  It   is  highly

recommended  that   you  upgrade   your  DOS   if

possible. For users with the  DOS burned into the

ROM of  the machine, you  may boot from  a system

floppy of a higher DOS system to compile scripts.

Problem:        When I transfer  a file, sometimes  letters (like

Flow or  Xoff) flash  in the  status line,  where

normally  the  status  of   the  Capture  Log  is

displayed.

Solution:       This is completely normal, and signifies a "flow"

control, or  a signal  to Telix  or the  modem to

slow down or  stop momentarily to save  the modem

from  being  flooded  with   too  much  data.  It

signifies that things are  in good working order.

After the end of the  connection, the Capture Log

status is displayed again.

Appendices  105

Problem:        How  do  I  make  Telix  operate  reliably  under

Microsoft Windows?

Solution:       TELIX.PIF  included  with  Telix   is  a  Program

Information File  for Windows  that should  allow

best operation of Telix  under Microsoft Windows.

Windows doesn't offer the  best of communications

handlers,  though,  and for  best  communications

results  under Windows,  we  recommend Telix  for

Windows, which  reliably supports  transfer rates

up to 115,200  bps under Windows and  offers many

additional features.

Problem:        My modem requires compatible software  to use the

MNP features of my modem, or it says it needs RPI

compatible software.Is Telix compatible for this?

Solution:       No, it  is not,  and there  is little  likelihood

that we will  support RPI or software  MNP in the

near or  distant  future. RPI  is  an attempt  by

Rockwell and the modem  manufacturers to create a

cheaper modem (by  about $5) by pushing  off some

of the hardware implementation  into software. We

disagree  with  this for  the  sole  reason  that

software cannot be as efficient as hardware (esp.

when coprocessed), and that these functions truly

belong on the hardware  for efficiency and speed.

Most comm  developers we know  feel the  same way

and without  our support  the manufacturers  will

have to go back to putting these functions on the

hardware - where they belong.

106  Appendices

Problem:        When I run  QDHost it says  Either the  upload or

download directory as defined  in the Host config

does not exist and then aborts. What now?

Solution:       If  you receive  this  message  when running  the

QDHost Mode then you need to do the following:

From Telix Terminal  Mode (the blank  screen that

you are at  after the opening screen  goes away),

press <ALT><G>  (or select  Script / eXecute from

the  menu) and  type  QDCONFIG. The  QDCONFIG.SLC

script must exist in the same directory as QDHOST

(i.e. in  the script  directory as  defined under

Configuration / Preferences / Filenames & Paths).

You will then  see a menu that  pops up something

like this:

A:   Level 1 password  :          pass1

B:   Level 2 password  :          pass2

C:   Remote Shell password :      shell

D:   Shut down host pass :        shut

E:   Host download directory:

C:\TELIX\HSTFILES\

F:   Host upload directory :

C:\TELIX\HSTFILES\

G:   Connection type :            Modem

H:   Modem locked at >= 9600:     No

I:   Exit without saving changes.

J:   Exit and save changes to disk.

The options E and F are the ones that need to be changed. You can

either  Exit without  saving and  then  do MKDIR  with the  above

paths:

MKDIR C:\TELIX\HSTFILES

or, better, is to change options E  and F above to paths that you

know  already  exist  (NEVER  set  these   equal  to  your  Telix

subdirectory!), and then Exit and save  changes to disk. For more

information concerning DOS paths, please consult your DOS manual.

Appendices  107

Problem:        Why does the estimated length  of a file transfer

fluctuate  for  some seconds  before  a  constant

value is displayed?

Solution:       Up to version 3.22, Telix estimated the length of

a file transfer by means of the bit rate reported

by  the modem  upon  successful connection  (e.g.

CONNECT  14400)  when dialing  from  the  Dialing

Directory) or  by means  of the  bit rate  set in

Telix when dialing manually  (with ATDT od ATDP).

However,  since  this  method   does  not  regard

factors like  line quality  and data  compression

and Telix had  to rely on a  correct modem result

code in the first case, Telix 3.50 now uses a new

method:  At the  beginning  of  a file  transfer,

Telix measures  the actual  number of  characters

transferred  per   second   and  calculates   the

estimated transfer  time from  this value.  Since

this value often fluctuates at the beginning of a

transfer,  Telix  measures   the  throughput  for

several seconds  and then  uses an  average value

for the  estimation. The estimated  transfer time

must not  be regarded as  an exact value,  but is

normally more exact  than an estimation  based on

the bit  rate reported  by the  modem or  the DTE

rate set in Telix.

Appendices  109

C    External Protocols

While Telix supports a very large number of protocols internally,

it also allows users to define up to five external protocol

implementations, for the utmost in flexibility.

External protocols are defined under

Configuration / Preferences / Protocol options. Basically, based

on this configuration, when the user selects a transfer with this

protocol, Telix will either run a specified DOS Batch file, or a

TELIX SALT script file.

Batch File Method

If a Batch file has been configured to be executed when the

external protocol is selected, TELIX will execute it and pass to

it 3 parameters (with the third sometimes being blank). The first

parameter is the baud rate. This is accessed within the DOS batch

file as %1. The second parameter is the comm port number. This is

accessed as %2. The third parameter is the reply the user gave

when asked what file(s) to transfer. If this is a download and

this protocol has been defined to not need a download name, this

parameter will be blank. If this is an upload via an external

protocol defined as an @ protocol (see Configuration, Protocols),

this parameter will be FILELIST, otherwise, will be the list of

files to send. This parameter is referred to as %3 in the Batch

file.

It is now the responsibility of the Batch file to call a driver

program for the protocol, using the supplied info. For example,

assuming Telix didn't have built-in Zmodem support, a batch file

called SZB.BAT could be defined to perform Zmodem uploads using

the commonly available DSZ program with the following command in

it.

DSZ port %2 speed %1 sz %3

This would tell DSZ what files to transfer, and also would

specify the proper baud rate and comm port.

110          Appendix: External Protocols

Script File Method

Alternately Telix can run a SALT script file when the user

selects an external protocol. The script should be compiled ahead

of time (as explained elsewhere in this manual). The user's reply

to the question of what files to transfer is stored in the system

variable _ext_filespec. The script file is free to implement the

protocol in whatever way it wants to, such as calling an external

driver program using the RUN function, or by actually performing

the transfer itself.

Appendices  111

D    File Formats

For programmers who wish to write Dialing Directory handling

utilities, a text file describing the Dialing Directory format

used by Telix is available in the Telix forum of the Support BBS.

112       Index

Index

14,400 bps ...................................................101

16550 UART ...........................................15, 61, 100

28,800 bps ...................................................101

7E1 ...........................................................15

8N1 ...........................................................15

Abort character ...............................................68

Aborted downloads .............................................59

Adding entries ................................................34

Alarm .........................................................59

ANSI ..........................................................78

ANSI BBS ......................................................77

ANSI.KEY .....................................................100

Answerback string .............................................58

ASCII .....................................................49, 73

ASCII transfers ...............................................68

AT ................................................62, 64, 89, 98

Auto baud detect ..............................................65

Auto-answer string ........................................29, 64

AVATAR ........................................................79

Backspace .................................................33, 56

BAT ...........................................................70

Batch file ...................................................109

Baud rate .....................................................15

BBS script ....................................................87

BIOS ...........................................................8

BIOS INT14 routines ...........................................11

Bit rate ......................................................15

Blank lines, expand ...........................................68

bps rate ......................................................15

Break signal ..................................................28

Busy signal ...................................................97

Capture file ..........................................17, 60, 66

Carriage Return .......................................16, 56, 69

Carrier Detect ................................17, 87, 90, 97, 98

Character pacing ..............................................69

Chat ..........................................................93

Chat mode .....................................................27

CIS Quick B ...................................................48

Clear screen ..................................................27

Clearing entries ..............................................34

Index  113

Color settings ................................................53

COM parameter ..................................................9

Comm port setup ...............................................74

Command entry .................................................12

Command line options ...........................................8

Command summary screen ........................................13

Command line options ..........................................14

COMMAND.COM ...............................................23, 99

Communications parameters .............................14, 55, 99

Communications port .......................................15, 74

Compiling Scripts .............................................81

Compuserve Quick B ........................................48, 58

COMSPEC .......................................................23

CONFIG.SYS .....................................................5

Configuration ..................................................6

Configuration file .....................................9, 10, 53

Connect string ................................................63

Connection type ...............................................88

Control character .............................................16

CR translation ................................................69

Crash recovery ................................................72

CRC errors ...................................................100

CS.EXE ........................................................82

CSS.EXE .......................................................83

CTS ...........................................................57

Customizing Telix .............................................53

Data compression .............................................102

Date format ...................................................60

DCE rate .....................................................101

Deleting entries ..............................................39

DESQview ...............................................8, 11, 54

Dial cancel string ............................................64

Dial time .....................................................64

Dialing .......................................................36

Dialing Directory ..........................................9, 31

Dialing Directory format .....................................111

Dialing Directory, creating ...................................40

Dialing Directory, loading ....................................40

Dialing Directory, printing ...................................41

Dialing Directory, sorting ....................................41

Dialing prefix ............................................33, 63

Dialing settings ..............................................62

Dialing suffix ................................................63

114       Index

Directory program .............................................67

Disk buffer ...................................................71

Disk drives ...................................................61

DOORWAY mode ..................................................29

DOORWAY.EXE ...................................................93

DOS command ...............................................23, 25

DOS functions .................................................24

DOS shell .....................................................23

Download .....................................43, 58, 59, 92, 103

Download directory ............................................66

DSR ...........................................................57

DTE rate .....................................................101

DTR ...................................................17, 57, 65

Echo ..................................................28, 56, 68

Editing an entry ..............................................34

Editing strings ...............................................15

Editor ................................................24, 25, 67

EMS ...........................................................61

Emulation .....................................................77

Ending a Telix session ........................................16

Enhanced keyboard .............................................60

Escape character ..............................................16

Estimated file transfer time .................................107

Executing a script ............................................83

Exiting Telix .................................................16

External protocols .......................................70, 109

File functions ................................................24

File transfer protocol ....................................43, 47

File transfer time ...........................................107

File transfers ................................................43

Filename guessing .............................................72

Filenames and Path Settings ...................................66

Files directory ...........................................24, 67

Finding an entry ..............................................35

Flow control ..................................................57

FOSSIL driver .................................................11

Garbage .......................................................99

General help ..................................................13

Goodbye .......................................................94

Handshaking ...................................................57

Hang-up ...................................16, 17, 59, 64, 65, 97

Hayes compatible ..............................................98

Help/Status screen ............................................13

Index  115

High bit ..............................................33, 56, 68

Host download directory .......................................88

Host mode .....................................................87

Host upload directory .........................................88

HOSTPLUS ......................................................87

Init string ...................................................62

Inserting blank entries .......................................39

INT14 parameter ................................................9

INT14/FOSSIL mode .........................................11, 74

Kermit ....................................................50, 74

KEY ...........................................................79

Keyboard commands .............................................95

Keyboard definitions ..........................................21

Language setup ................................................75

Learning a script .............................................84

Left-handed mouse .............................................12

LF translation ................................................69

Line Feed .....................................16, 28, 33, 56, 69

Line pacing ...................................................68

List dial .....................................................36

LNG .......................................................10, 75

Local echo ............................................28, 56, 68

Locked modem rate .............................................89

Log ...................................................18, 60, 66

LOGO.MSG ......................................................90

Long distance codes .......................................38, 40

Machine Requirements ...........................................4

Macros ........................................................21

Manual dial ...................................................36

Manual dialing ................................................38

Menu bar ..................................................12, 61

Menu line .....................................................61

Miscellaneous Options .........................................29

MNP ..........................................................105

Modem initialization string ...............................29, 62

Modem settings ................................................62

Modem sharing program .........................................11

Modem7 ........................................................50

MODEMCFG.EXE ...................................................6

Mouse support .................................................10

Moving entries ................................................41

Multi-tasking ....................................10, 11, 98, 100

Network operation .............................................11

116       Index

No connect strings ............................................63

NOMOUSE parameter .............................................10

Online .........................................................8

Output string translation .....................................16

Pace character ................................................68

Pacing ....................................................68, 69

Parity ........................................................15

Password ..............................................33, 87, 88

Path Settings .................................................66

Pause .........................................................16

Pause capturing ...............................................17

Preferences menu ..............................................53

Prefix ........................................................33

Printer log ...................................................17

Protocol ..............................................33, 43, 47

Protocol options ..............................................70

QDCONFIG .........................................87, 90, 92, 106

QDHost ...............................................87, 97, 106

QDHOST.LOG ....................................................90

Queue redial ..................................................37

Quick dialing bar .........................................38, 59

Quiet mode .....................................................9

Receiving files ...............................................43

Recording a script ............................................84

Redial ........................................................37

Redial pause ..................................................65

Registration Card ..............................................1

RPI ..........................................................105

RSHELL.BAT ....................................................93

RTS ....................................................8, 57, 58

SALT ..........................................................81

Screen image ..............................................25, 67

Screen settings ...............................................53

Screen size ...................................................29

Script ........................................9, 70, 81, 84, 104

Script directory ..............................................66

Script learning ...............................................84

Script, executing .............................................83

Script, linked ................................................32

Scripts, creating and compiling ...............................81

Scroll-back ...............................................20, 60

SEAlink .......................................................49

Searching for a string ........................................20

Index  117

Sending files .................................................45

Sending words/characters with mouse ...........................13

Serial Number ..................................................x

SHARE.EXE .....................................................11

Sharing Violation Errors ......................................11

Shell .........................................................92

SIMPLE ........................................................83

Sound .........................................................59

Speed .........................................................15

Standard communications mode ...................................9

Start-up directory .............................................5

Startup script ................................................66

Status line ...............................................14, 56

Strings, entering and editing .................................15

Support ....................................See Technical Support

Swap ..........................................................61

System Requirements ............................................4

Technical Support .............................................ix

Australia ....................................................xi

CompuServe Forum .............................................ix

Germany ......................................................xi

Great Britain ................................................xi

Internet Email ...............................................ix

Outside the United States ....................................xi

Support BBS ..................................................ix

Voice Support .................................................x

Teletype ......................................................77

Telink ........................................................50

Telix for Windows ............................................105

TELIX*.LNG ....................................................10

TELIX.CAP .....................................................66

TELIX.CNF ...........................................7, 9, 10, 53

TELIX.FON ..................................................9, 31

TELIX.IMG .................................................25, 67

TELIX.KEY .....................................................21

TELIX.MDM ......................................................6

TELIX.PIF ....................................................105

TELIX.SWP .....................................................61

TELIX.USE .................................................18, 66

Terminal ..............................................32, 55, 77

Terminal mode .................................................12

Terminal options ..............................................55

Time format ...................................................60

118       Index

TOFF parameter ................................................10

Transfer protocol .........................................43, 47

Translate table ...............................................26

Trigger string ................................................71

Troubleshooting ...............................................97

TTY ...........................................................77

UART ....................................................100, 102

Upload ...........................................45, 61, 92, 103

Upload directory ..............................................66

Usage log .............................................18, 60, 66

V.42bis ......................................................102

VT102 .........................................................78

VT52 ..........................................................79

WELCOME.MSG ...................................................91

Windows .............................................98, 100, 105

XLT ...........................................................27

Xmodem ....................................................47, 48

Xmodem timing .................................................71

XMS ...........................................................61

XON/XOFF ......................................................57

Ymodem ........................................................49

Ymodem-g ......................................................50

Zmodem ............................................49, 58, 72, 73

Zmodem auto-downloads .........................................43